READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - Extent of Work: The extent and location of the “Demolition” Work is indicated on the drawings. The Work includes the requirements for the removal, wholly or in part and satisfactory disposal of all buildings, utilities, fences, pavements, curbs, structures, light fixtures, or other features identified within these specifications. The Demolition Work is included on conduits, wires and other obstructions which are designated to be demolished on the drawings. The drawings should be used for guidance only and to indicate typical general construction features of the various types of structures and is not to be construed as definitive or adequate to supplant the actual on-site inspection by the Contractor.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - U.S. Department of Labor Occupational Safety & Health Administration

      - OSHA Standard 1926.850(a), Preparatory Operations

    - King County burning of debris

    - METRO

    - Puget Sound Energy

    - City of Seattle material transfer tax

    - Seattle City Light

    - Telephone Company

    - Puget Sound Clean Air Agency

  - SUBMITTALS

    - Submit materials data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Demolition plan.

      - Proposed landfills and recyclers.

- MATERIALS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - PROJECT INFORMATION

  - PREPARATION FOR MATERIALS

  - FABRICATION, PRODUCTION, & SUPPLY OF MATERIALS

  - MATERIAL REQUIREMENTS

  - MATERIAL HANDLING, DELIVERY, & STORAGE

  - DELIVERABLES

  - QUALITY ASSURANCE

- EXECUTION

  - PROJECT INFORMATION

    - The Contractor represents that it has visited the site to become familiar with the quantity and character of all materials to be demolished. The Contractor agrees that the premises were made available prior to deadline for submission of bids for whatever inspection and tests the Contractor deemed appropriate in preparation of bids.

  - PREPARATION FOR EXECUTION OF WORK

    - The Contractor shall submit to the Port of Seattle a demolition plan that at a minimum addresses the following:

    - Products that are required to accomplish, or to be incorporated into, the Work of this section shall be as selected by the Contractor, subject to the approval of the Port Representative.

    - Salvage Items

      - Steel or wooden structures, utilities, or equipment to be salvaged for the Port of Seattle shall be carefully dismantled without unnecessary damage. [Steel] [Wood] [Stone] members shall be match-marked.

        - Worker safety.

        - Protection of the public.

        - Protection of workers or other persons in floors below, areas surrounding, and demolition site.

        - Work sequence.

        - Means and methods to minimize waste and maximum salvage.

        - Disposal procedures.

        - Protection of the environment.

      - All material designated to be salvaged for the Port of Seattle shall be delivered and stored as specified in the salvage list:

        - Salvage List:	To be stored at:

          - [___]	[___]

        - Salvage List:	To be delivered to:

          - [___]	[___]

    - Demolition Items

      - Structures to be demolished:

        - [___]

      - Utilities to be demolished:

        - [___]

      - Improvements to be demolished:

        - [___]

Advisory Memo to recipient is in order prior to preparation of salvage list.

  - EXECUTION OF WORK

  - DEMOLITION OF STRUCTURES

    - Completely remove and dispose of legally off-site, designated buildings, foundations, structures, fences and other obstructions. All pavements, sidewalks and curbs designated for removal shall be broken up, loaded and disposed of by the Contractor. Care shall be taken in removing the pavement, so that damage does not occur to the existing pavement which is to remain in place and that all removals are accomplished by making a neat full depth vertical saw cut at the boundaries of the area to be removed. Adjacent materials designated to remain that are damaged by the Contractor due to his operations shall be replaced at no additional cost to the Port.

    - Blasting or other special operations necessary for the removal of an existing structure or obstruction shall be subject to the approval of the Port Representative.

    - Unless otherwise noted on the plans, the Contractor shall have the following options for removing piling and bulkhead. Option 2 below, requires the Contractor to verify new pile locations to avoid conflicts.

      - Pulling piling.

      - Breaking off piling at the mud line.

  - DEMOLITION OF UTILITIES

    - Well in advance of the demolition of any utilities, the Contractor shall advise the appropriate agency of the impending action and arrange with each utility, [Seattle City Light], [METRO], [Puget Sound Energy], [telephone company], [Fiber], [Seattle Public Utility],[Sound Transit] for all Work required by that utility under this contract. Special conditions required by the utility shall be the sole responsibility of the Contractor. Contact the utilities for bond requirements, if any, prior to bid.

      - Piping: Remove all piping in the demolition area, including underground piping, exposed piping, or piping fixed to timber structures.

      - Electrical and Telephone Items: Remove electrical conduit, fixtures and equipment from the demolition area.

      - Water Lines: Remove and cap water and other utility lines in accordance with applicable codes.

      - Storm Drains: Remove catch basins in the demolition area, including outfall piping through the bulkhead. Remove city outfall structures and trim pipe to clear new construction.

  - DISPOSAL

    - General: All materials, except those indicated as Port of Seattle salvage, and except those materials containing substances classified as hazardous or potentially hazardous by local, state or Federal regulating agencies, shall upon their demolition become the property of the Contractor. All such material, including those containing hazardous or potentially hazardous substances shall be removed and promptly disposed of legally away from the site and on property not owned by the Port of Seattle, except as otherwise provided in these specifications. No material shall be disposed of in adjoining waterways or in the fill. Burning of materials in these areas falls under the jurisdiction of the King County regulations and is generally forbidden under all circumstances.

    - Cleanup: After removal of structures, buildings and foundations, clean and grade the area. There shall be no debris, rubble, or litter left at the site from any of the demolition operations and the site shall be clean.

    - The Port of Seattle encourages the salvage and recycling of materials from demolished structures. The Contractor shall salvage or recycle, in an acceptable manner to environmental agencies and the Port of Seattle, at his option any of the materials designated for disposal.

    - Non-salvageable or non-recyclable demolition, contaminated soils and creosote debris shall be transported to a Port approved lined landfill with a Leachate Collection System [and shall be subject to the City of Seattle's material transfer tax.]

  - DELIVERABLES

  - QUALITY ASSURANCE

- MEASUREMENT AND PAYMENT

  - MEASUREMENT

    - [Removal of Concrete] will be measured in place as the actual number of square yards of concrete removed, regardless of the thickness.

    - [Removal and Salvage of Timber] will be measured by the thousand board feet at the salvage stockpile site using the nominal thickness and width and the actual length of the individual pieces removed.

    - [Removal of Railroad Tracks] will be measured by the linear foot in place along the centerline of tracks, spurs, turnouts, or cross-overs which have been removed and [salvaged] [disposed of] or as directed.

    - [Remaining Demolition] [Demolition] will be measured as a unit and shall include all materials, structures, buildings, fences, utilities, pavements, railroads, sidewalks, curbs, piers, or wharves not included in unit bid items above.

  - PAYMENT

    - Payment for [___] will be made at the contract [unit] [lump sum] price as stated in the Schedule of Prices and will be full compensation for furnishing all labor, equipment and incidentals required to accomplish the demolition as specified.

- End of Section

- Revision History:

- 05/01/2014 Conversion to 2004 CSI Numbering System

- 10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2