READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Finish Carpentry” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, & REFERENCES

    - None.

  - SUBMITTALS

    - Submit materials data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Samples

  - QUALITY ASSURANCE

    - Qualifications of Workers: Molding, trim and other finish materials shall be cut and fitted by or under the direction of journeyman finish carpenters who are thoroughly trained and experienced in the skills required. Workers must be completely familiar with the materials involved and the installation recommendations of the manufacturer.

    - No allowance will be made for lack of skill on the part of workers. Finish carpentry Work not properly installed will be rejected.

  - PRODUCT HANDLING

    - Protection: Protect finish carpentry Work before, during and after installation by whatever means are necessary to prevent damage by the Work of this or other sections.

    - Replacement: In the event of damage, immediately make all repairs and replacements to the approval of the Engineer at no additional cost to the Port.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - GENERAL

    - See Section 06 05 00 - Lumber and Timber.

- EXECUTION

  - PREPARATORY REVIEW

    - Prior to all Work of this section, carefully inspect the installed Work of all other trades and verify that all such Work is complete to the point where the Work of this section may properly commence and be completed in strict accordance with the original design.

    - In the event of discrepancy, immediately notify the Engineer for resolution before proceeding.

  - WORKMANSHIP

    - General: All finish carpentry joints, seams, or fittings shall be true, tight and well fastened. All members shall be assembled in accordance with the approved drawings.

    - Jointing:

      - Make all joints to conceal shrinkage; miter all exterior corners; cope all interior corners; miter or scarf all end-to-end joints.

      - Install all trim and molding in pieces as long as possible, jointing only over backing or other solid support.

    - Fastening: Use galvanized fasteners only when using waterborne salt-treated materials.

      - Install all items straight, level, plumb, properly oriented and firmly anchored in place. Coordinate with all other trades to ensure proper placement of all blocking and backing required.

      - Nail interior trim with finish nails of the proper size; drill as required to prevent splitting.

      - Nail exterior trim with galvanized finish nails of the proper size; drill as required to prevent splitting. Make all joints to exclude water and set in waterproof glue or in caulking compound as described in Section 07 92 00 - Joint Sealers of these specifications.

      - Set all nails for putty on exposed finish Work.

      - Screws may be started by driving except on exposed finish Work or where splitting may occur. Pre-drill screw holes on all exposed finish Work. Turn screws into final position on all Work, using care not to damage the screw head or driving recess.

    - Floors under Resilient Covering: Lay the underlayment in as large sheets as practicable over 3/4-pound unsaturated deadening felt. Fit the sheets tight and nail them to the joists below with L head-cut flooring nails set below the surface with the special tool used for that purpose. Space nails at four inch centers around the edges and at eight inch centers along the intermediate bearings. Fill the joints between sheets and nailheads with wood putty. After the wood putty has set, remove the overwood at the joints by machine sanding to leave a smooth and even surface.

    - Siding: Install where indicated on the drawings, in even courses of as long lengths as practicable, with joints at courses staggered.

    - Wood Door and Relight Frames: Wood door and relight frames shall be 1-5/8 inches thick, solid, double-rabbeted except where detailed otherwise. House the heads into jambs and set the joints of exterior frames in an approved sealant.

    - Screening at Vents: Install the aluminum wire mesh insect screen with a lap of at least two inches at all openings. Stretch the material taut and tack it securely in place using galvanized staples.

  - INSTALLATION OF OTHER ITEMS

    - Install all other items to be installed under this section in strict accordance with the drawings and the manufacturers’ recommendations. Set the items true, straight, plumb and level, and anchored them securely for the intended use.

  - FINISHING

    - Sandpaper all finish wood surfaces to produce a uniformly smooth surface. No imperfections, such as hammer marks or coarse grain sandpaper marks, will be accepted. Touch up or refurbish pre-finished or factory-finished surfaces, as necessary, according to the manufacturers' recommendations.

  - CLEANUP

    - Keep the Work area in a neat, safe, orderly condition at all times, free from an accumulation of sawdust and debris. Leave the area broom-clean upon completion.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2