READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

The extent and location of “Cementitious Fireproofing” Work is shown in the Contract Documents. Use A if asbestos abatement is involved.

    - Provide fireproofing for steel wherever asbestos fireproofing was removed. Fireproofing shall provide the same or better protection as the existing asbestos fireproofing. The fireproofing shall be dyed the specified color so the new material can be easily identified from any remaining asbestos containing fireproofing.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - 2012 IBC

    - ASTM E605 - 93(2011): Standard Test Methods for Thickness and Density of Sprayed Fire-Resistive Material (SFRM) Applied to Structural Members

    - ASTM E736 - 00(2011): Standard Test Method for Cohesion/Adhesion of Sprayed Fire-Resistive Materials Applied to Structural Members

    - ASTM E759 - 92(2011): Standard Test Method for Effect of Deflection on Sprayed Fire-Resistive Material Applied to Structural Members

    - ASTM E1513 - 93(2011): Standard Practice for Application of Sprayed Fire-Resistive Materials (SFRMs)

    - ASTM E760 - 92(2011): Standard Test Method for Effect of Impact on Bonding of Sprayed Fire-Resistive Material Applied to Structural Members

    - ASTM E761 - 92(2011): Standard Test Method for Compressive Strength of Sprayed Fire-Resistive Material Applied to Structural Members

    - U.L. labels

    - U.L. test data

    - UL 263: Standard for Fire Tests of Building Construction and Materials

  - SUBMITTAL REQUIREMENTS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Submit manufacturer’s directions for application.

  - QUALITY ASSURANCE

    - The applicator shall be approved by the material manufacturer and certification of such approval shall be furnished to the Engineer.

    - The applicator shall examine the substrate and the conditions under which the insulation Work is to be performed and notify the Contractor in writing of any unsatisfactory conditions. Do not proceed with the fireproofing Work until unsatisfactory conditions have been corrected in a manner acceptable to the applicator.

    - The Contractor shall provide protective enclosures to prevent spray of material outside building and to prevent overspray of material to any adjacent surfaces not requiring fireproofing

  - HANDLING OF MATERIALS

    - Deliver in original containers bearing U.L. labels.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - SPRAYED FIREPROOFING (CEMENTITIOUS)

    - Materials

      - A factory-compound mixture of synthetic aggregate and gypsum sprayed in-place to form a monolithic, dust-free fire protection course of uniform thickness. Minimum dry density shall be 15 pounds per cubic foot per U.L. test data.

Competition waiver to be submitted to CPO for W.R. Grace Company to be the only approved product for the Port.

    - Fire Protection Listing and Manufacturer

      - The fireproofing material shall be cementitious kind only as listed in the Underwriter’s Laboratories Building Materials List Guide No. 40 U 18.3 and Retro-Guard RG as manufactured by W.R. Grace Company, No Equal, formulated without asbestos and dyed blue.

    - Performance Standard

      - Bond Strength - 200 lbs/s.f. (min. ave) in accordance with ASTM - E736

      - Compression - 1200 lbs/s.f. (min ave) in accordance with ASTM - E761

      - Bond Impact - No cracking or delamination in accordance with ASTM - E760

      - Deflection - No cracking or delamination in accordance with ASTM - E759

- EXECUTION

  - INSPECTION

    - Applicator shall examine the areas and conditions under which fireproofing coatings are to be applied and notify the Contractor in writing of conditions detrimental to the proper and timely completion of the Work. Do not proceed with the Work until unsatisfactory conditions have been corrected in a manner acceptable to the applicator. Starting of fireproofing coatings Work will be construed as the applicator’s acceptance of the surfaces within any particular area. Do not apply coatings over dirt, rust, scale, grease, moisture, or fireproofing conditions unacceptable in manufacturer’s instructions.

  - SURFACE PREPARATION

    - Perform preparation and cleaning procedures in compliance with the coating manufacturer’s instructions for the particular substrate conditions.

    - Remove items which are not to be coated, or provide surface-applied protection prior to the surface preparation and coating operations.

    - Following coating completion in each space or area, reinstall removed items, using workmen skilled in the trades involved. Clean surfaces to be coated before applying coatings or surface treatments in accordance with manufacturer’s instructions.

  - MATERIALS’ PREPARATION

    - Mix and prepare materials in compliance with the manufacturer’s directions. Store materials in tightly-covered containers. Maintain containers used in storage, mixing and application of coating in a clean condition, free of foreign materials and residue. Stir all materials before application to produce a mixture of uniform density and as required during the application of the materials.

  - APPLICATION OF SPRAYED FIREPROOFING (CEMENTITIOUS)

    - General

      - Application shall be in accordance with manufacturer’s printed instructions and in accordance with fire test report as to thickness and density.

    - Protection of Completed Fireproofing

      - At the completion of the installation of the sprayed fireproofing, fireproofing material shall be protected against any scraping or removing except as required for the installation of additional hangers and clips. At any location where additional hangers and clips are required, fireproofing shall be patched to its original required thickness.

  - RATINGS REQUIRED

    - Apply the fireproofing to a thickness to provide a three-hour rating, unless indicated otherwise.

  - INSPECTION AND REPAIR

    - Inspect surfaces for complete coverage. After completion of other Work, repair all damaged areas.

  - TESTING

    - Special Inspection Required:

      - Thickness and Density per ASTM E605: Provide the necessary equipment required to test new fireproofing for proper installation thickness. Test random locations, as selected by the Engineer.

      - Adhesion/Cohesion Test per ASTM E763:

  - CLEAN-UP

    - After approval of fireproofing, remove equipment and clean floors and walls of material deposits and overspray.

  - CERTIFICATE

    - As part of the submittal process, prior to application of fireproofing, furnish a manufacturer’s certificate stating that the fireproofing materials contain no asbestos.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2