READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Wood doors” Work is shown in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - Standards:

      - General: Comply with the requirements of the following standards unless otherwise indicated.

      - Non-Fire-Rated Wood Doors: NWWDA Industry Standard I.S. 1 “Wood Flush Doors” of the National Wood Windows and Doors Association.

      - Fire-Rated Wood Doors: Where fire-resistant classifications are shown or scheduled for plastic-faced wood door assemblies, provide doors which comply with the requirements of NFPA No. 80 “Standard for Fire Doors and Windows” and which have been tested and rated with single-point hardware by Underwriters Laboratories. Provide a UL label on each door and panel.

    - Asbestos Containing Material (ACM) Verification:

      - All doors shall be ACM free and shall bear “ACM Free” label.

      - Provide Material Safety Data Sheet (MSDS) certifying ACM free products

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Product Data: For each type of door include details of core and edge construction, and trim for openings.

      - Shop Drawings: Indicate location, size, and hand of each door; elevation of each kind of door; construction details not covered in Product Data; location and extent of hardware blocking; and other pertinent data.

        - Indicate dimensions and locations of mortises and holes for hardware.

        - Indicate dimensions and locations of cutouts.

        - Indicate fire ratings for fire doors.

      - Samples for verification: As follows:

        - Corner sections of doors approximately 8 x 10 inches with door faces specified finish.

  - SPECIAL WARRANTIES

    - Before final inspection, furnish the manufacturer’s warranty for each door for the life of the original installation.

  - PRODUCT DELIVERY, STORAGE AND HANDLING

    - General: Protect wood doors during transit storage and handling to prevent damage, soiling and deterioration. Comply with manufacturer's instructions and with the "On-Site-Care" requirements of NWWDA pamphlet "Care and Finishing of Wood Doors.”

    - Fire-Rated Doors: Comply with manufacturer's instructions for precautions required to prevent moisture deterioration of fire-retardant salts.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - GENERAL REQUIREMENTS FOR DOORS

    - Materials and Fabrication: Wherever flush wood doors are shown or scheduled, provide door construction complying with referenced standards and the requirements which follow.

    - Face Panels: Fabricate with 2 or more plies and with face veneer of the species and cut specified, Premium [Good] Grade (NWWDA). Provide same material on both faces of individual doors.

      - Face Veneer: [White oak, plain sliced] [Rotary Birch] [___].

      - Veneer Match:

      - Book match, center match with an even number of pieces of veneer across door face and with a joint occurring on the centerline of the door with the same figure and grain on each side of the centerline.

    - Exposed Edges: Door manufacturer's standard close-grain hardwood, for paint finish.

OR

    - Solid hard wood of same species as face veneers.

    - Manufacturers: Provide doors as manufactured by Cal-Wood, Eggers, Paine Lumber Co., Algoma Hardwoods, Weyerhaeuser, Vancouver Door Co., Or Approved Equal

  - FIRE-RATED FLUSH WOOD DOORS

    - Where wood doors with fire-resistance classifications are shown or scheduled provide face veneer and finish to match non-fire-rated solid core wood doors in the same area of the building. Door construction shall comply with the requirements previously specified under "Quality Assurance" for fire-rated wood doors.

  - SOLID CORE FLUSH WOOD DOORS

    - Core Construction: Solid wood-block, wood particleboard, or mineral with wood lock blocks, as required by door manufacturer to comply with the specified guarantee period.

    - Adhesive Type: Provide Type II water-resistant bond; except where "Exterior Door" is shown or scheduled for interior location, provide doors with Type I bond.

  - HOLLOW CORE FLUSH WOOD DOORS

    - Core Construction: Interlocking horizontal and vertical wood strip grid, as required by door manufacturer to comply with the specified guarantee period.

    - Adhesive: Type II adhesive bond.

  - CUT-OUTS

    - Louvers: Door shall be cut for louvers for the insert type where indicated.

    - Glazing: Doors shall be cut for glazing where indicated. Provide metal glazing stops, factory primed.

  - SHOP FINISH

    - General: Pre-finish all wood doors at the door factory. Comply with recommendation of AWI for factory finishing of doors, including final sanding with No. 6/0 finishing paper immediately before the first application of finishing materials.

    - Finish: Provide finishes to match with and comply with AWI Finish System No. 4, Premium Grade (Vinyl Catalysed).

OR

    - Finish: Provide finishes to match with and comply with manufacturer’s applicable standard finish specifications for [mfr/finish].

- EXECUTION

  - INSTALLATION

    - Conditioning: Condition doors to average prevailing humidity in installation area prior to hanging.

    - Hardware: For installation see Section 08 70 00 - Hardware.

    - Manufacturer's Instructions: Install wood doors in accordance with manufacturer's instructions and as shown.

    - Clearances: For non-fire doors provide clearances of: 1/8-inch at jambs and heads; 1/8-inch at meeting stiles for pairs of doors; and 1/2-inch from bottom of door to top of decorative floor finish or covering, except where threshold is shown or scheduled provide 1/4-inch clearance from bottom of door to top of threshold.

    - Fire-Rated Doors: Install in corresponding fire-rated frames in accordance with the requirements of NFPA No. 80. Provide clearances complying with the limitations of the authority having jurisdiction.

  - ADJUST AND CLEAN

    - Operation: Re-hang or replace doors which do not swing or operate freely, as directed by the Engineer.

    - Protection of Completed Work: Installer shall advise Contractor of proper procedures required for protection of installed wood doors from damage or deterioration until acceptance of the Work.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2