READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Special Door” Work is indicated in the Contract Documents. The Work includes the requirements for providing special doors, frames, hardware and miscellaneous appurtenances.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - None.

  - SUBMITTALS

    - Submit materials data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Shop Drawings: Shop drawings shall indicate all fabrication and installation details.

      - Materials List: Submit a complete list of associated materials and equipment provided with the special doors.

  - QUALITY ASSURANCE

    - Qualification of Installers: Doors shall be installed by personnel who are thoroughly trained and experienced in the installation of the specified doors. The final installation shall be approved by the authorized distributor prior to final inspection and approved by the Engineer. Insulating doors shall have a U rating of under 0.2.

    - Codes and Standards: Where noted in the schedules, the doors shall bear the Underwriters Laboratories label for fire rating.

  - PRODUCT HANDLING

    - Package, handle and protect doors to prevent damage to the unit until approved during final inspection. Damage, if any, shall be repaired or parts replaced at no additional cost to the Port.

    - Protect hardware or other door components as above.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - SECTIONAL DOORS

    - Manufacturer: Crawford, Clopay, Raynor, Hercules, Kinnear, Or Approved Equal.

    - Type and Construction: Doors shall be of sectional, upward-acting, box-type, galvanized steel sections and shall be prepainted.

    - Operation: Operating hardware shall be torsion-spring, high-lift, engineered for size and weight of door. Provide vertical-lift hardware for doors less than ten feet high.

    - Track: Shall be two inch, hot-dipped galvanized steel, securely fastened to jambs by continuous angles and inclined to ensure weather tight closure. Place horizontal track as close as possible to the ceiling. Hinged and roller brackets shall be of 14 gauge minimum, hot-dipped galvanized steel, formed to provide mounting for rollers to edges of doors and designed to provide tight contact between sections. Rollers shall have not less than ten 1/4 inch diameter hardened steel ball bearings with hardened inner and outer race.

    - Counterbalance: The weight of the door shall be counterbalanced by means of oil-tempered helical torsion springs with grooved cable drums mounted on steel shafts extended the full width of the opening. Lift cables shall be galvanized steel attached securely to corner lift brackets.

    - Corner Lift Brackets: Provide corner lift brackets designed for the size and weight of the door.

    - Components: Miscellaneous hardware components except castings, springs, spring shaft, and cable drum, shall be zinc or cadmium plated. Locks shall be inside/outside, single-bar, with key cylinder and night latch. See Section 08 70 00 – Hardware.

    - Reinforcing: Provide according to standard engineering practices.

  - STEEL ROLLING DOORS

    - Manufacturer: Atlas, Cookson, Kinnear, Mahon, Wilson, Or Approved Equal.

    - Type and Construction: Standard non-labeled service door with curtain slats to resist wind pressure of 20 psf; provide wind locks as required. The bottom bar shall be two steel angles with standard weather-stripping per manufacturer.

    - Guides: Structural steel shapes of size sufficient to form proper slot for curtain; provide anchors for securing to wall.

    - Operation, Manual: Manual operation standard with manufacturer, hand chain type. The curtain shall be manually raisable after automatic action without rewinding.

    - Operation (electric):

      - General: Curtain opened and closed by electric motor; emergency operation by hand chain.

      - Motor: General-purpose, open-type, ball bearing, standard N.E.M.A. frame; type to suit installation conditions and available electric current. Capacity to operate curtain at 0.67 feet per second without overloading.

      - Controls: Furnish limit switches, safety stop, governor and other control equipment required. Control switches, 3-bottom type, on interior. Cooperate with electrician in adjusting doors for proper electric operation.

    - Counterbalance: Curtain coiled on steel pipe, balanced by helical steel spring inside; provide means for accurate and easy adjustment of spring tension.

    - Hood shall be 24 gauge, galvanized sheet steel. The hood shall tightly fit the contour of the end plates.

    - Lock shall be a deadbolt locking from the inside only. See Section 08 70 00 – Hardware.

    - Surface Treatment: Surfaces of curtain and hood shall be galvanized and bonderized, Or Approved Equal.

    - Insulation: Each slat shall be formed enclosing polyurethane insulation.

- EXECUTION

  - INSTALLATION

    - Install all special doors in strict accordance with the original design, all applicable codes and regulations, the approved shop drawings and the manufacturer’s recommendations. Firmly anchor or tighten all components for hard use.

  - TOUCH-UP

    - Upon completion of the installation, touch up all scuffs and abrasions with the manufacturer’s specified shop-coat primer.

  - INSTRUCTIONS

    - Supply the Engineer with all special operating or maintenance manuals.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2