READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Aluminum Window” Work is indicated in the Contract Documents. The Work includes the requirements for providing the aluminum windows and all necessary appurtenances.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - None.

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Shop Drawings: submit complete shop drawings.

      - Manufacturer’s Recommendations: Submit current installation recommendations with shop drawings specified.

  - QUALITY ASSURANCE

    - Standard Specifications: Comply with applicable requirements of AAMA 101.

  - PRODUCT HANDLING

    - Package, handle and protect windows and components to prevent damage to the units until approved during final inspection. Damage, if any, shall be repaired or parts replaced at no additional cost to the Port.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - GENERAL

    - All aluminum windows, components and hardware shall be manufactured or distributed by a single company unless specified otherwise.

  - NON-OPERABLE FIXED SASH

    - Approved Manufacturers:

      - Herzog Aluminum Co., “Series 4000T”, thermally broken, 2-inch frame depth.

      - Kawneer Co., “Sealair 8225-L”, thermally broken, 2.25-inch frame depth.

      - Wausau Metals Corp., “Classic 25”, thermally broken, 2.5-inch frame depth.

      - Or Approved Equal.

    - Performance: Aluminum fixed (F) windows shall conform to AAMA 101 [F-R15] [F-LC25] [F-C30] [F-HC40] [F-AW40] type, non-operable glazed frame, complete with provisions for re-glazing in the field.

    - Materials: All frame and panel members shall be constructed of heavy-walled, extruded aluminum shapes of alloy Type 6063-T5.

    - Finish: All exposed portions of the aluminum sash shall be given a [0.4-mil] [0.7-mil] anodic oxide “Commercial [Clear] [Bronze] [Light] [Medium] [Dark] Anodize” coating.

    - Structural Thermal Barrier: Poured in place two-part polyurethane.

    - Construction: All corners of frame members shall be mitered and welded or tenon and riveted. Type and size as indicated on the drawings or schedule.

    - Glazing: See Section 08 80 00 - Glazing of these specifications.

OR

    - Glazing: Panels shall be glazed with [glazing stops] [extruded vinyl glazing channels]. Channels shall accommodate glass of 3/16 inch, 1/4-inch, or 5/8 inch thick insulated glass, fabricated in accordance with requirements of Section 08 80 00 - Glazing of these specifications.

When other types of sash are required--Casement, Single Hung, Double Hung, Rolling, etc.--follow the format above adding those requirements for Hardware, Screens, Weather-stripping, Ventilators as they apply.

- EXECUTION

  - INSTALLATION

    - Set and anchor windows level, plumb and in true alignment in strict accordance with the manufacturer’s approved shop drawings.

  - CLEANUP

    - After installation, remove the protective coatings, if any, and clean the aluminum surfaces in accordance with the manufacturer’s directions. Leave the windows and frames clean and free from defects.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2