READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Gypsum Base for Veneer Plaster” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - ASTM C588 gypsum base sheets

    - Gypsum Association GA-150 “Application of Gypsum Base for Veneer Plastering”

    - Underwriters Laboratories tested and listed

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

  - QUALITY ASSURANCE

    - Codes and Standards: Comply with applicable requirements of governing codes and authorities, and unless otherwise indicated or specified, comply with the applicable requirements of GA-150 of the Gypsum Association - “Application of Gypsum Base for Veneer Plastering.”

    - Fire-Resistance Ratings: Comply with fire-resistance ratings as required by governing authorities and codes. Provide materials, accessories, and application procedures, which have been tested and listed by Underwriters Laboratories for the type of construction indicated.

    - Installer Requirements: Comply with the requirements of the Gypsum Association.

  - PRODUCT HANDLING

    - Delivery and Storage of Materials: Protect materials from weather and store in dry, ventilated space, off the ground, away from damp or sweating surfaces and under cover. Deliver in unopened bundles, fully identified with name, brand, type and grade.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MATERIALS

    - Gypsum Base for Veneer Plaster: Provide gypsum base sheets complying with ASTM C588, manufacturer’s standard sheet size, of the type and thickness indicated on the drawings.

    - Accessories for Gypsum Base Sheets: Provide fasteners and other support accessories for the type and sizes recommended by the base sheet manufacturer for the application shown.

- EXECUTION

  - INSTALLATION

    - Manufacturer’s Instructions: Except as otherwise indicated, install gypsum base sheets and accessories in accordance with the lath manufacturer’s printed instructions.

    - Apply base sheets with face sides exposed. Butt and fit edges together for moderate contact, but do not force into place. Form solid external corners. When cutting, cut both front and back paper facings. Maintain close tolerances for openings at electrical outlets, fixtures, piping and other penetrations and obstructions in the lathing Work.

    - Fasteners: Do not locate fasteners closer than 3/8 inch from ends or edges of sheets. Drive screws with a power screwdriver as recommended by the base sheets manufacturer.

    - Wall Applications: Stagger joints between gypsum base sheets on opposite sides of partitions. Do not locate vertical joints within eight inches from corners of door frames and other openings.

      - For ceiling heights of 8’-2” and less, apply gypsum base vertically or horizontally at the Contractor’s option. For ceiling heights greater than 8’-2” and walls less than 4’-0” wide, apply gypsum base vertically with all joints occurring over studs. Use floor-to-ceiling length base sheets for vertical applications. For horizontal applications, use the maximum practical length of base sheets required to minimize end joints and stagger end joints over studs.

      - Fasten gypsum base to metal studs or metal furring with self-tapping screws.

      - Comply with the manufacturer’s instructions for fastening, but do not exceed a spacing of 12 inches on center for screws; except, in vertical applications, space screws along edge joints not more than nine inches on center.

    - Ceiling Application: Provide a single layer in the maximum practical lengths which minimize end joints.

    - On metal furring channels, apply the length of the sheet at right angles to the channel, with end joints over the channel. Fasten with self-tapping screws in accordance with the manufacturer's instructions, but at not less than 12 inches on center at all furring channels.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2