READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Resilient Wall Base” Work and all associated materials is as shown in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - None.

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Submit samples of the type of wall base material specified for approval of color, texture and pattern.

      - Employ at least one person who shall be present at all times during the execution of this portion of the Work and who shall be thoroughly trained and experienced in the installation of the wall base specified, and who shall direct all Work performed under this section.

  - PRODUCT HANDLING

    - Delivery and Storage of Materials: Deliver all materials to the job site in unopened containers bearing the manufacturers’ labels; do not open containers or remove labels until the Engineer inspects and approves.

    - Precautions: Maintain the temperature of resilient wall base materials at not less than 70°F for 48 hours before and after laying.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - BASE

Competition waiver to be submitted to CPO for Burke to be the only approved product for the Port.

    - Provide new rubber base four inches and six inches high, as indicated on the drawings, top set, manufactured by Burke, No Equal. Color must be approved by the Engineer.

    - At carpeted floors, provide new rubber base four inches high carpet base, manufactured by Burke, No Equal. Color must be approved by the Engineer.

  - OTHER MATERIALS

    - General: Provide accessory materials, equipments, tools and methods as required for a complete installation.

    - Primers, Adhesives and Sealers: Provide water-resistant types made or recommended by the wall base manufacturer;

- EXECUTION

  - INSTALLATION

    - General:

      - Install all materials after the concrete is completely dry, in strict accordance with the manufacturer’s recommendation; install wall base by cementing directly to substrate.

      - Condition of Surfaces: Verify the condition of substrates prior to installation; installation of wall base will be construed as acceptance of all conditions of the installation, including temperatures, moisture content, cleanliness and general condition of the substrate. Should surfaces be deemed unacceptable for the installation of wall base, notify and await the direction of the t Engineer.

    - Installation of Base: Lay out base material and cut to lengths, carefully match edges at all seams, or double-cut adjoining lengths. Arrange the joints so as to be practically invisible. Adhere base materials securely to the substrate, laying the material in straight lines with top edges level and true. Form internal and external corners.

  - FINISHING

    - After the base has set sufficiently, wash with neutral cleaner; apply two coats of non-slip wax, type as recommended by the base materials manufacturer. Buff to a smooth, dull-gloss finish. Leave surfaces clean and smooth, free from waves, buckles, cracks, projecting edges and other defects.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2