READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Tab) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

If Work of this project is to be conducted at Sea-Tac Airport in exterior weather-exposed locations and over impervious surfaces and includes copper, galvanized steel, or zinc in any other form, obtain concurrence from Port Environmental with the specified approach (coating) to prevent contamination of storm water entering the storm water system at the Sea-Tac Airport. Alternatively, specify other materials as noted in this section. Revision of this section includes updating CSI Master Format Section numbers and titles to conform to the latest edition in use by the Port.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Paints and Coatings” Work is indicated in the Contract Documents.

    - Surfaces Not To Be Painted

      - Unless specifically specified otherwise, the following surfaces are not to be painted:

        - Exposed finish metals (aluminum, brass, bronze, stainless steel, copper, chrome).

        - Sealant, interior and exterior.

        - Ceramic tile, brick.

        - Exterior concrete (both precast and cast-in-place).

        - Glass and plastic laminate.

        - Resilient floor coverings, bases, mats.

        - Acoustic tile, lay-in panels, metal suspension systems.

        - Items having a complete factory finish, except items built into surfaces which have painted finish.

        - Permanently concealed surfaces need not be painted, except for prime coats on metal and millwork.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - All semi-gloss

    - Federal Specification Standards materials

    - Manual of Uniform Traffic Control Devices paragraph 3B-18 layout

    - MPI appendix

    - MPI exceptions

    - MPI linked cross-references

    - Master Paint’s Institute Manual additional approved manufacturers - paint

    - Master Painter’s Institute Manual listed type brand and manufacture

    - Master Painter’s Institute materials not specifically noted

    - MPI preparing painting and finishing schedules

    - MPI references

    - MPI selection references

    - Master Painter’s Institute Inc. (MPI) standard specifications 90.00

    - MPI supplementary references

    - MPI surfaces not scheduled

    - SSPC SP-1 ferrous metal surfaces

    - State Specification Standards materials

    - Exterior 1-A, 1-B, 1-C, 1-D, 1-E, 1-F

    - Exterior 2-A, 2-B, 2-C, 2-D, 2-E

    - Exterior 3-A, 3-B

    - Exterior 4-A, 4-B, 4-C

    - Exterior 5-A, 5-B, 5-C

    - Exterior 6-A, 6-B, 6-C, 6-D

    - Exterior 7-A, 7-B

    - Exterior 8-A, 8-B, 8-C

    - Exterior 9-A

    - Exterior 10-A

    - Exterior 11-A, 11-B, 11-C

    - Exterior 12-A

    - Exterior 13-A, 13-B, 13-C, 13-D

    - Exterior 14-A, 14-B, 14-C

    - Exterior 15-A, 15-B

    - Exterior 16-A

    - Exterior 17-A

    - Exterior 18-A, 18-B

    - Interior 1-A, 1-B, 1-C, 1-D, 1-E, 1-F, 1-G, 1-H, 1-I, 1-J, 1-K, 1-L, 1-M, 1-N, 1-O, 1-P, 1-Q, 1-R, 1-S, 1-T, 1-U, 1-V, 1-W, 1-X Interior 2-A, 2-B, 2-C, 2-D, 2-E, 2-F, 2-G, 2-H, 2-I, 2-J, 2-K, 2-L, 2-M, 2-N, 2-O

    - Interior 3-A, 3-B, 3-C, 3-D, 3-E, 3-F, 3-G, 3-H

    - Interior 4-A, 4-B, 4-C

    - Interior 5-A, 5-B, 5-C, 5-D, 5-E, 5-F

    - Interior 6-A, 6-B, 6-C, 6-D

    - Interior 7-A, 7-B, 7-C, 7-D, 7-E, 7-F

    - Interior 8-A, 8-B

    - Interior 9-A, 9-B

    - Interior 10-A, 10-B, 10-C

    - Interior 11-A, 11-B, 11-C

    - Interior 12-A, 12-B, 12-C, 12-D

    - INT 12-A, 14-A, 3-B, 6-B all semi-gloss from 04/97

    - Interior 13-A

    - Interior 14-A

    - Interior 15-A, 15-B

    - Interior 16-A, 16-B

    - Interior 17-A

    - Interior 18-A, 18-B

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittal shall include the following:

      - Submit color samples of all paints allowing ample time for consideration before the material to be painted is delivered or ready for painting. Where the same color stain is applied to different kinds of wood, carefully sample and test mixing proportions to arrive at the same relative appearance.

  - QUALITY ASSURANCE

    - Standard Specifications: “Master Painter’s Institute” (MPI), latest edition, published by the Master Painters Institute Inc., Las Vegas, Nevada (604) 298-7578. The words "Specifying Authority" as used in the Standard Specifications Section shall mean the Port of Seattle.

    - Application: Shall be by an experienced painter or a painting firm employing experienced personnel.

    - Manufacturers’ Specifications, Directions and Recommendations: Conform to manufacturers’ specifications, directions and recommendations for best results in the use of each of their products for each condition. If results are at variance with these specifications, report the discrepancy to the Resident Engineer for decision.

    - Layout: The location, lettering size, and style of the surface regulatory signs shall be in accordance with the applicable portions of paragraph 3B-18 of the “Manual of Uniform Traffic Control Devices” as published by the U.S. Department of Transportation, Federal Highway Administration.

  - PRODUCT HANDLING

    - Delivery and Storage: Deliver paint materials in unbroken, unopened containers bearing the manufacturers’ labels. Do not open containers or remove labels until the Engineer inspects and approves. Store materials in a dry location where the indicated ambient temperature of storage is not less than 50°F.

    - Precautions: Take extraordinary care to prevent fire, open containers or inflammable materials only as needed. Keep rubbing cloths and oily rags in tightly closed metal containers, or remove from the site daily. Benzene, gasoline and distillate will not be permitted on the job site.

    - Protection: Care shall be exercised in the handling of painting materials to ensure that this Work and the Work of other trades are not damaged before, during, or after the installation.

    - Replacements: Repair or replace damaged Work, if any, as necessary to the approval of the Engineer at no additional cost to the Port.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MATERIALS

    - General: Unless otherwise specified, furnish paint, varnish, stain, enamel, lacquer, fillers and related products for prime and intermediate and finish coats, of a type brand and manufacture listed in Master Painter’s Institute Manual, latest edition, factory-labeled for positive identification. Materials not specifically noted in the Master Painter’s Institute , and required for the Work such as linseed oil, shellac, thinners or other materials required for the Work, shall be of quality not less than required by applicable published Federal or State Specification Standards, and as manufactured by approved firms.

    - Supplemental Specifications

      - Additional Approved Manufacturers - Paint: Unless otherwise specified, in addition to approved manufacturers' listed in the Master Paint’s Institute Manual, paint materials may also be as manufactured by Pratt & Lambert, provided best grade primers and finish coats manufactured by them (and recommended by them for the particular surfaces to be painted) are used.

Add any other exception to MPI or any supplementary references not covered by MPI.

    - Materials selected for coating systems for each type of surface shall be the product of a single manufacturer.

    - All paint materials shall be the respective equivalent, in the opinion of the Engineer, to the several types of materials specified. Deliver all materials to the job site in the original, unbroken containers, bearing the manufacturers’ labels indicating the contents and directions for use, storage and handling.

    - Materials not specifically noted but required for the Work, such as linseed oil, shellac, thinners, or other materials, shall be the product of the approved paint manufacturer.

  - MIXING

    - Mix paint products according to the manufacturer’s printed directions. Do not adulterate in any manner except upon specific approval of, and in the presence of, the Engineer.

  - COLOR SELECTION

    - The color selection will be made by the Engineer basically from Port of Seattle Standard Colors, with additions from other standards as designated by the Engineer.

  - IDENTIFICATION

    - The manufacturer’s identification numbers and specifications listed are for the purpose of indicating the type and quality of paint product desired for the purpose indicated.

  - SUBSTITUTIONS

    - If the Contractor desires to use an alternate manufacturer’s materials or methods of application, these shall be submitted in writing to the Engineer for review and approval prior to procurement of material. Substantiating technical data is required. Approval will not be granted unless, in the opinion of the Engineer, the quality of finished construction will be equal to that intended. Substitutions shall be at no additional cost to the Port.

- EXECUTION

  - GENERAL

    - Apply the paints in accordance with the manufacturer’s recommendations as to the application, weather and temperature conditions. Provide “highest” quality workmanship performed to the Engineer’s satisfaction. Use clean equipment and brushes when applying paint; spread paint materials evenly, without runs, sags, laps, or brush marks, without variations in color, texture, or sheen and without “holidays.” Vary colors or sheens between coats and apply all coats to uniform thicknesses. Cut sharp lines against glass, other materials and different colors. Recoat suction spots in the first coat on drywall as necessary to produce uniformity of color and gloss. Refinish any Work judged defective at no additional cost to the Port of Seattle; repair all Work damaged during the progress of the construction. Leave finished surfaces clean, completely covered, uniform in appearance and satisfactory to the Engineer.

  - SURFACE PREPARATION

    - Provide means to control odors so that they do not affect or impact any Port, tenant, or passenger operations.

    - Ensure any generation of dust is contained and does not affect or impact any Port, tenant, or passenger operations.

    - Clean all surfaces thoroughly, removing all rust, mill scale, fabrication films, dust, dirt and other foreign matter from surfaces. Apply film to completely dry surfaces.

    - Galvanized Metal: Thoroughly clean surfaces, wiping with mineral spirits or xylol. If silicone surface treatments have been applied in the fabrication shop, use xylol; remove silicates or similar surface treatments and deposits of “white rust” by sanding or other approved abrasive methods. Thoroughly clean and rinse contaminants from surfaces.

    - Ferrous Metal Surfaces: Thoroughly clean using mineral spirits, xylol, or toluol in accordance with SSPC SP-1. Take care to ensure that adequate ventilation is provided at all times when using solvents. Carefully rinse and clean surfaces before applying paint.

    - Gypsum Wallboard: Inspect surfaces for proper joint treatment and correct as required; sand all joint cement smooth before applying any paint; prime joint cement areas before applying the first coat or prime coat of finish material.

    - Wood: Before applying the finish to wood surfaces, sand and smooth for even, uniform finish application; use No. 80 grit-1/0 sandpaper for rough sanding and shaping; use No. 100 or 120 grit-2/0 or 3/0 for preparatory sanding for softwoods; use No. 220 grit-6/0 for finish sanding of softwoods. Seal all sap or knots, fill holes and cracks with plastic wood or putty and sand smooth. Remove all traces of oil, wax, grease, adhesive, or other foreign matter.

    - Condition of Surfaces: Inspect and approve conditions of substrate surfaces scheduled to receive paint; notify the Engineer of any surfaces unsuitable for application as specified. The application of a paint finish constitutes an acceptance of the surface as suitable, unless directed to proceed in writing by the Engineer. Do no outside Work during wet or freezing weather, or until surfaces have thoroughly dried from the effect of such weather; see that proper temperatures and ventilation are maintained for all interior Work.

    - Mixing and Thinning: Mix and thin paint products in strict accordance with the manufacturer’s directions; mix and thin other materials in accordance with the “best” trade practices as approved.

    - Sheen: Where gloss or sheen is specified or is listed as standard for approval for the Project, the terms refer to tested luster, shine, or sheen of the dry film. For purposes of this specification, luster, shine, or sheen is defined as follows when tested with a 60-degree gloss-meter; flat, 10-degree gloss or less; eggshell 11- to 19-degree gloss; satin, 20- to 30-degree gloss; semi gloss, 31- to 74-degree gloss; and gloss, 75-degree gloss or more.

  - APPLICATION

    - Number of Coats: As specified in paragraph 3.06 for each type of finish. On shop-primed Work, an additional prime coat is not intended.

    - Thickness of Coats: Use ample undiluted materials; apply in a uniform thickness over entire areas. Do not exceed the manufacturer's recommended spreading rate per gallon.

    - Colors of Coats: Tint prime coats if necessary to obtain uniform finish coats. Vary color between coats; the final coat shall exactly match approved samples.

    - Approval of Successive Coats: Obtain the Engineer’s approval of each coat before the succeeding coat is applied; if this approval is not obtained, the Engineer reserves the right to require an additional coat.

    - Prime and finish door jambs by using brush or spray. Rollers or other methods are not acceptable.

  - MECHANICAL OR ELECTRICAL EQUIPMENT

    - Apply primer and two finish coats as specified for the appropriate metal surface, including ductwork, according to the finish schedule.

  - PROTECTION OF FINISHED WORK

    - Use tarpaulins or drop cloths when working above or adjacent to completed Work; clean all paint splatters and stains from finished surfaces. Protect all Work from dust and insects.

  - PAINTING AND FINISHING SCHEDULES

    - General: Painting and Finishing Titles and Code Numbers -References are from the MPI unless otherwise indicated. With linked cross-references in the MPI, they indicate coating, grades, manufacturers, and the like. Selections relate to surface, type, coating, grade, and named products and their manufacturers listed in the MPI. Provide Work on the drawings as scheduled, specified, and a directed. Consult MPI for surfaces not scheduled and follow Engineer’s directions.

    - Exterior Painting and Finishing Schedule: Provide the Work as shown on the drawings and as scheduled following:

Following is example of schedule to be prepared for the project. See Appendix at end of section for MPI Paint/Finishing Systems. Also have MPI available for reference when preparing Painting and Finishing Schedules.

      - Flashing and Sheet Metal: EXT. 14-A Semi-gloss.

      - Roof Hatches: EXT. 12-A Semi-gloss.

      - Metal Doors and Frames, Window Frames: EXT 14-A Semi-gloss.

      - Coiling Doors: EXT 14-A Semi-gloss.

      - Metal Fabrications: EXT 14-A Semi-gloss.

    - Interior Painting and Finishing Schedule: Provide the Work as shown on the drawings and as scheduled following:

Following is example of schedule to be prepared for the project.

See Appendix at end of section for MPI Paint/Finishing Systems. Also have MPI available for reference when preparing Painting and Finishing Schedules.

      - Concrete Unit Masonry: INT. 6-B Semi-gloss

      - Metal Doors and Frames, Window Frames: INT 14-A Semi-gloss.

      - Coiling Doors: INT 14-A Semi-gloss.

      - Metal Fabrications: INT 14-A Semi-gloss.

      - Gypsum Board: INT. 3-B Semi-gloss for walls, Eggshell for ceilings.

      - Mechanical Equipment: INT. 12-A Semi-gloss.

      - Electrical Equipment: INT. 12-A Semi-gloss.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2

# APPENDIX - EXTERIOR PAINTING

## Woodwork (*Fences, *siding, joists, plywood, partitions, underside decking, pressure treated lumber, etc. - Other than trim and glued-laminated members.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 1-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 1-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 1-C** ##### Stained Finish |  |  |  |
| 1 | Exterior Wood Stain |  |  |
| 2 | Exterior Wood Stain |  |  |
| #### Exterior 1-D** ##### Pressure Treated Lumber |  |  |  |
| 1 | Clears, Stains |  |  |
| 2 | Clears, Stains |  |  |
| #### Exterior 1-E*** ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Recycled Primer | Recycled Primer |  |
| 2 | Recycled Finish | Recycled Finish |  |
| 3 |  | Recycled Finish |  |
| #### Exterior 1-F*** ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Recycled Primer | Recycled Finish |  |
| 2 | Recycled Finish | Recycled Finish |  |
| 3 |  | Recycled Primer |  |

*Fence and siding includes fir, cedar, redwood, cypress, pine, etc

**Exterior 1-C - Coat surfaces and edges prior to installation. Check coating and plywood manufacturer's recommendations.

**Exterior 1-D - Refer to individual manufacturer's recommendations.

**Fire Retardant Treatment may be required

***Standards for application per manufacturer’s recommendation. Colors are limited due to recycling process.

Chapter 5 - Page 3 Revised 04/97

## Wood Trim (Doors, door frames, window sash and frames, fascia, battens, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 2-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 2-B ##### Stained Finish (Gloss as Selected) |  |  |  |
| 1 | Wood Stain |  |  |
| 2 | Wood Stain |  |  |
| #### Exterior 2-C* ##### Natural Finish (Varnish) |  |  |  |
| 1 | Varnish (reduced 25%) | Varnish (reduced 25%) |  |
| 2 | Varnish | Varnish |  |
| 3 |  | Varnish |  |
| #### Exterior 2-D* ##### Natural Finish |  |  |  |
| 1 | Clear Wood Finish |  |  |
| 2 | Clear Wood Finish |  |  |
| #### Exterior 2-E ##### Paint Finish Waterborne |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Exteriors 2-B, 2-C and 2-D

Coat surfaces and edges prior to installation, and check coating and plywood manufacturer's recommendations.

*Exteriors 2-B and 2-D Custom recommended, check manufacturers recommendations

Chapter 5 - Page 4 Revised 04/97

## Wood Floors (Stairs, steps and thresholds, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 3-A* ##### Enamel Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel (reduced 25%) | Floor Enamel (reduced 25%) |  |
| 2 | Floor Enamel | Floor Enamel |  |
| #### Exterior 3-B ** ##### Enamel Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel |  |  |
| 2 | Floor Enamel |  |  |

*Exterior 3-A Check coatings and wood products manufacturer for application recommendations.

**Exterior 3-B Only Custom Grade System recommended. Check manufacturer's recommendation.

Chapter 5 - Page 5 Revised 04/97

## Wood Shingles and Shakes (Siding, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 4-A* ##### Standard Shingle Stain Finish (Pigmented Stains) |  |  |  |
| 1 | Shingle Stain |  |  |
| 2 | Shingle Stain |  |  |
| #### Exterior 4-B* ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 4-C* ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Exterior 4-A, B & C Check coatings and wood products manufacturer for application recommendations.

*Exterior 4-A Custom Grade system recommended. Check manufacturer's recommendations

## Exterior Glued-Laminated Members

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 5-A* ##### Stained Protective Finish |  |  |  |
| 1 | Wood Stain |  |  |
| 2 | Wood Stain |  |  |
| #### Exterior 5-B* ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 5-C* ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Exterior 5 A, B & C Check coatings and wood products manufacturer for application recommendations.

*Exterior 5 A only Custom Grade system recommended. Check manufacturer's recommendation.

## Concrete, Concrete Masonry Units, Masonry, Stucco, Stone and Brick*

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 6-A* ##### Pigmented Coating Solvent Base (Gloss as Selected) (Except Cement Masonry Units) |  |  |  |
| 1a | Cementitious Surface Conditioner, Solvent Base / Waterborne | Cementitious Surface Conditioner, Solvent Base / Waterborne |  |
| 1b | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 6-B* ##### Pigmented Coating - Concrete Masonry Units Solvent Base / Waterborne (Gloss as Selected) |  |  |  |
| 1a | Cementitious Surface Conditioner, Solvent Base / Waterborne | Cementitious Surface Conditioner, Solvent Base / Waterborne |  |
| 1b | Block Filler Solvent Base / Waterborne Primer | Block Filler Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 6-C* ##### Elastomeric Finish Clear or Pigmented Solvent Base / Waterborne |  |  |  |
|  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. Use only manufacturer's complete system. |  |  |
| #### Exterior 6-D* ##### Clear or Pigmented Solvent Base / Waterborne |  |  |  |
| 1 | Clear or Pigmented |  |  |
| 2 | Clear or Pigmented |  |  |

*Refer to Chapter 4 for clear dampproofing and water repellent coatings.

Specify pigmented aggregate coatings in Section 09 97 00 - Special Coatings ; follow manufacturer's recommendations.

*Exterior 6 A, B, C & D Check coatings and concrete, masonry, stucco and drivit manufacturers for application recommendations.

*Certain substrates may require surface conditioners.

*Exterior 6-D A Custom Grade system recommended. Check manufacturer's recommendations.

*Moisture content reading must be taken so proper coating can be applied. Check manufacturer's recommendation.

## Glass Fiber Reinforced Concrete

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 7-A ##### Pigmented Coating Solvent Base |  |  |  |
| 1 | Alkali-Resistant Primer | Alkali-Resistant Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 7-B ##### Pigmented Coating Waterborne |  |  |  |
| 1 | Alkali-Resistant Primer | Alkali-Resistant Primer | 2.1, 2.2 |
| 2 | Finish | Finish | 6 |
| 3 |  | Finish | 6 |

*Moisture content reading must be taken so proper primer can be applied. Check manufacturer's recommendation.

## Asphalt and Asphaltic Concrete (Driveways, floors, parking lines, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| Exterior 8-A ##### Zone Marking (Non-Sealed) Solvent Base |  |  |  |
| 1 |  | Traffic Paint |  |
| 2 | Traffic Paint | Traffic Paint |  |
| #### Exterior 8-B* ##### Zone Marking (Seal Coated) Waterborne |  |  |  |
| 1 | Traffic Paint | Traffic Paint |  |
| 2 | Traffic Paint | Traffic Paint |  |
| #### Exterior 8-C ##### Zone Marking (Non-Sealed) Waterborne |  |  |  |
| 1 | Traffic Paint | Traffic Paint |  |
| 2 | Traffic Paint | Traffic Paint |  |

*Waterborne systems recommended. Check manufacturer's recommendations.

## Asphalt, Impregnated or Coated Areas (Insulation, pipes, siding, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 9-A ##### Painted Finish Waterborne (For prevention of asphalt bleed-through) |  |  |  |
| 1 | Finish | Finish |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

## Asbestos-Cement Siding

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 10-A ##### Asbestos -Cement Siding |  |  |  |
|  |  | Special precautions and application procedures must be followed i.e., .encapsulation or removal will be required. Call the Asbestos Removal toll free number 1-800-552-3565 for current regulations or questions. |  |

## Ferrous Metal - (Light and ordinary exposures) (Not galvanized)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 11-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 11-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 11-C ##### Aluminum Paint Finish |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

## Steel - High Heat Exterior (boilers, breeching, furnace, fronts, pipe lines, flues, heat exchangers, turbines, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 12-A ##### Temperature Range 800 Degrees & Above |  |  |  |
|  |  | Proper coats on bare metal in accordance with manufacturer’s directions. |  |

## Galvanized (Zinc-Coated) Metal

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 13-A* ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Primer |  |
| 2 |  | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 13-B* ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Finish | Finish |  |
| 3 | Finish | Finish |  |
| #### Exterior 13-C* ##### Aluminum Paint Finish |  |  |  |
| 1 |  | Pretreatment Primer / Exterior Galvanized Primer |  |
| 2 |  | Aluminum Paint |  |
| 3 |  | Aluminum Paint |  |
| #### Exterior 13-D** ##### Black Finish (Unexposed) |  |  |  |
| 1 | Bituminous Enamel |  |  |

*A Premium Grade system is recommended only. Check manufacturer's recommendation.

**Apply according to manufacturer’s recommendation.

## Aluminum

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 14-A ##### Painted Finish (Exposed) Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Aluminum Primer | Aluminum Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 14-B ##### Painted Finish (Exposed) Waterborne (Gloss as Selected) |  |  |  |
| 1 | Aluminum Primer | Aluminum Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 14-C* ##### Black Finish (Unexposed) |  |  |  |
| 1 | Bituminous Enamel |  |  |

*Apply according to manufacturer's recommendation.

## Hardboards, Overlays, Composites, etc.

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 15-A* ##### (Unprimed) (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Solvent Base / Waterborne Finish | Solvent Base / Waterborne Finish |  |
| 3 |  | Solvent Base / Waterborne Finish |  |
| #### Exterior 15-B* ##### (Primed) (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Solvent Base / Waterborne Finish | Solvent Base / Waterborne Finish |  |
| 3 |  | Solvent Base / Waterborne Finish |  |

*Exterior 17-A and 17-B

Check lumber composites and oriented strandboard manufacturer's recommendations for product instructions and application methods.

## Plastic Piping (ABS, PVC, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior16-A* ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | N/A* |  |
| 2 |  | Finish |  |
| 3 |  | Finish |  |

*Exterior 18-A - Piping must be cleaned with lacquer thinner to eliminate all excursion contaminants. Use premium grade system only.

*Primer is not necessary; if warranted, contact manufacturer for recommendations.

## Precoated Siding (aluminum, galvanized) and Colored Vinyl Siding

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior17-A* ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Primer | Primer |  |
| 2 | Exterior 100% Acrylic Latex | Exterior 100% Acrylic Latex |  |
| 3 |  | Exterior 100% Acrylic Latex |  |

*Exterior 19-A - Surfaces may be chalking from exposure and deterioration; consult manufacturer for special cleaning procedure coating additives to ensure adhesion.

*Some waterborne acrylics may be self-priming; consult manufacturer.

## Special Coating System - Graffiti (Sacrificial & Non-Sacrificial)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Exterior 18-A* ##### Graffiti Coat Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Finish | Finish |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Exterior 18-B* ##### Graffiti Coat Waterborne (Gloss as Selected) |  |  |  |
| 1 | Finish | Finish |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Check manufacturer's recommendation for coatings system and graffiti removal.

# APPENDIX - INTERIOR PAINTING

## Wood (Standing and running trim, doors, door frames, exposed joists, paneling, plywood, window frames and sash, wood deck ceilings, cabinets, partitions, glued-laminated members, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 1-A Solvent Base Finish (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Undercoat (tinted) |  |
| 3 |  | Finish |  |
| #### Interior 1-B ##### Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base / Waterborne Primer | Solvent Base / Waterborne Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 1-C* ##### Solvent Base Finish on Stained Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Stain | Wood Stain |  |
| 2 | Sanding Sealer | Sanding Sealer |  |
| 3 | Varnish | Varnish |  |
| 4 |  | Varnish |  |
| #### Interior 1-D* ##### Solvent Base Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Sanding Sealer | Sanding Sealer |  |
| 2 | Varnish | Varnish |  |
| 3 |  | Varnish |  |
| #### Interior 1-E* ##### Solvent Base Finish on Stained Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Stain | Wood Stain |  |
| 2 | Sanding Sealer | Sanding Sealer |  |
| 3 | Varnish | Varnish |  |
| 4 |  | Varnish |  |
| #### Interior 1-F* ##### Solvent Base Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Sanding Sealer | Sanding Sealer |  |
| 2 | Varnish | Varnish |  |
| 3 |  | Varnish |  |
| #### Interior 1-G* ##### Solvent Base Finish on Stained Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Stain | Stain |  |
| 2 | Sanding Sealer | Sanding Sealer |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| 4 | Clear or Pigmented | Clear or Pigmented |  |

*For even stain color, certain woods require application of a pre-stain sealer under the stain; consult manufacturer.

*Manufacturer's may require a complete solvent base or waterborne system, check with manufacturer for recommendation.

## Wood (continued)

| #### FINISH SYSTEM CODE Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 1-H* Solvent Base Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Clear Lacquer Sealer Coat | Clear Lacquer Sealer Coat |  |
| 2 | Clear or Pigmented | Clear or Pigmented |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-I* ##### Solvent Base Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler (Eliminate 1st coat if grain not to be filled) | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 | Wood Stain (If wood to be filled and stained, first two coats can be combined by using a filler-stain) | Wood Stain (If wood to be filled and stained, first two coats can be combined by using a filler-stain) |  |
| 3 | Sanding Sealer | Sanding Sealer |  |
| 4 | Varnish | Varnish |  |
| 5 |  | Varnish |  |
| #### Interior 1-J* ##### Waterborne Finish on Stained Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Stain | Stain |  |
| 2 | Sanding Sealer | Sanding Sealer |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| 4 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-K* ##### Waterborne Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Clear Lacquer Sealer Coat | Clear Lacquer Sealer Coat |  |
| 2 | Clear or Pigmented | Clear or Pigmented |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-L* ##### Waterborne Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler (Eliminate 1st coat if grain not to be filled) | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 | Wood Stain (If wood is to be filled and stained, first two coats can be combined by using a filler stain.) | Wood Stain (If wood is to be filled and stained, first two coats can be combined by using a filler-stain.) |  |
| 3 | Sanding Sealer | Sanding Sealer |  |
| 4 | Varnish | Varnish |  |
| 5 |  | Varnish |  |

*Use only Premium Grade on surfaces subject to water, steam, or possible staining such as kitchen, utility, vanity, lab woodwork.

*For even stain color, certain woods require application of a pre-stain sealer under the stain; consult manufacturer.

*Manufacturer's may require a complete solvent base or waterborne system, check with manufacturer for recommendation.

## Wood (continued)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 1-M* ##### Solvent Base Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler (Eliminate 1st coat if grain not to be filled) | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  |  |  |
| 3 | Sanding Sealer | Sanding Sealer |  |
| 4 | Varnish | Varnish |  |
| 5 | Varnish | Varnish |  |
| #### Interior 1-N* ##### Solvent Base Finish on Stained Open Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler or Stain (Eliminate filler if grain not to be filled, and use stain) | Wood Filler or Stain (Eliminate filler if grain not to be filled, and use stain) |  |
| 2 | Lacquer Sanding Sealer | Lacquer Sanding Sealer |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| 4 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-O* ##### Solvent Base Finish on Natural Open Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Lacquer Sanding Sealer | Lacquer Sanding Sealer |  |
| 2 | Clear or Pigmented | Clear or Pigmented |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-P* ##### Waterborne Finish on Natural Close-Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler (Eliminate 1st coat if grain not to be filled) | Filler (Eliminate 1st coat if grain not to Wood be filled) |  |
| 2 |  | Sanding Sealer |  |
| 3 | Sanding Sealer | Varnish |  |
| 4 | Varnish | Varnish |  |
| 5 | Varnish | Varnish |  |
| #### Interior 1-Q* ##### Waterborne Finish on Stained Open Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Wood Filler or Stain (Eliminate filler if grain not to be filled, and use stain) | Wood Filler or Stain (Eliminate filler if grain not to be filled, and use stain) |  |
| 2 | Lacquer Sanding Sealer | Lacquer Sanding Sealer |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| 4 | Clear or Pigmented | Clear or Pigmented |  |

*Use only Premium Grade on surfaces subject to water, steam, and possible staining such as in kitchens, utility areas, and on lab work.

*Manufacturer's may require a complete solvent base or waterborne system, check with manufacturer for recommendation.

## Wood (continued)

| #### FINISH SYSTEM CODE Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 1-R* ##### Waterborne Finish on Natural Open Grain Woods (Gloss as Selected) |  |  |  |
| 1 | Lacquer Sanding Sealer | Lacquer Sanding Sealer |  |
| 2 | Clear or Pigmented | Clear or Pigmented |  |
| 3 | Clear or Pigmented | Clear or Pigmented |  |
| #### Interior 1-S ##### Oil Finish for Teak |  |  |  |
|  |  | Use only one manufacturer's oil finish products and follow manufacturer's recommendations. |  |
| #### Interior 1-T ##### Oil Resin Sealer Finish on Open and Close-Grain Woods |  |  |  |
|  |  | Use only one manufacturer's oil finish products and follow manufacturer's recommendations. |  |
| #### Interior 1-U ##### Fire Retardant Finish |  |  |  |
|  |  | Follow manufacturer's recommendations. Check local code requirements. |  |
| #### Interior 1-V ##### Elastomeric Finish on Plywood |  |  |  |
|  |  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. |  |
| #### Interior 1-W ##### Multi-Colored Coatings |  |  |  |
|  |  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. |  |
| #### Interior 1-X ##### Drywall Finish |  |  |  |
|  |  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. |  |

*Use only Premium Grade on surfaces subject to water, steam, and possible staining such as in kitchens, utility areas, and on lab work.

*Manufacturer's may require a complete solvent base or waterborne system, check with manufacturer for recommendation.

## Wood Floors and Stairs

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 2-A* ##### Natural Finish on Close-Grain Woods Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Floor Finish |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| #### Interior 2-B* ##### Stained Finish on Close-Grain Woods Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Wood Stain |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| #### Interior 2-C* ##### Natural Finish on Open Grain Woods Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| #### Interior 2-D* ##### Natural Finish on Close-Grain Woods Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Floor Finish |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| #### Interior 2-E* ##### Stained Finish on Close-Grain Woods Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Wood Stain |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| #### Interior 2-F* ##### Natural Finish on Open Grain Woods Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Floor Finish |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |

*Custom Grade systems not recommended on Floors and Stairs.

## Wood Floors and Stairs (Continued)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 2-G ##### Stained Finish on Open Grain Woods Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Wood Stain (If wood to be filled and stained, first two coats can be combined by using a filler-stain) |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| 5 |  | Floor Finish |  |
| #### Interior 2-H ##### Stained Finish on Open Grain Woods Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Wood Stain (Note: If wood to be filled and stained, first two coats can be combined by using a filler-stain) |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| #### Interior 2-I* ##### Opaque Enamel Finish (Close or Open Grain) Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Floor Enamel Finish |  |
| 2 |  | Floor Enamel Finish |  |
| 3 |  | Floor Enamel Finish |  |
| #### Interior 2-J* ##### Opaque Enamel Finish (Close or Open Grain) Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Floor Enamel Finish |  |
| 2 |  | Floor Enamel Finish |  |
| 3 |  | Floor Enamel Finish |  |

*Custom Grade systems not recommended on Floors and Stairs.

## Wood Floors and Stairs (Continued)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 2-K** ##### Gymnasium Marking Lines (Over the full floor finish system) Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel Finish |  |  |
| 2 | Floor Enamel Finish |  |  |
| #### Interior 2-L* ##### Stained Finish on Open Grain Woods Solvent Base (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Wood Stain (Note: If wood to be filled and stained, first two coats can be combined by using a filler-stain) |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| 5 |  | Floor Finish |  |
| #### Interior 2-M* ##### Stained Finish on Open Grain Woods Waterborne (Gloss as Selected) |  |  |  |
| 1 |  | Wood Filler (Eliminate 1st coat if grain not to be filled) |  |
| 2 |  | Wood Stain (Note: If wood to be filled and stained, first two coats can be combined by using a filler-stain) |  |
| 3 |  | Floor Finish |  |
| 4 |  | Floor Finish |  |
| #### Interior 2-N** ##### Gymnasium Marking Lines (Over the full floor finish system) Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel Finish |  |  |
| 2 | Floor Enamel Finish |  |  |
| #### Interior 2-O** ##### Gymnasium Marking Lines (Over the full floor finish system) Waterborne (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel Finish |  |  |
| 2 | Floor Enamel Finish |  |  |

*Custom Grade systems not recommended on Floors and Stairs.

**Only Custom Grade system recommended.

Revised 04/97 Chapter 6 - Page

## Gypsum Wallboard, Veneer (thin wall plaster)*, and Plaster Walls*and Ceilings

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 3-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne Primer Sealer* | Solvent Base or Waterborne Primer Sealer* |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 3-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne Primer Sealer* | Solvent Base or Waterborne Primer Sealer* |  |
| 2 | Stipple Paint | Finish |  |
| 3 |  | Finish |  |
| #### Interior 3-C ##### Painted Stipple Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne Primer Sealer* | Solvent Base or Waterborne Primer Sealer* |  |
| 2 | Finish | Stipple Paint |  |
| 3 |  | Finish |  |
| #### Interior 3-D ##### Painted Stipple Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne Primer Sealer* | Solvent Base or Waterborne Primer Sealer* |  |
| 2 | Stipple Paint | Stipple Paint |  |
| 3 |  | Finish |  |
| #### Interior 3-E* ##### Textured Finish (Gloss as Selected) |  |  |  |
| 1 | Primer Sealer | Primer Sealer |  |
| 2 | Texture Coat | Texture Coat |  |
| 3 | Interior Solvent Base or | Solvent Base / Waterborne Finish |  |
| 4 | Waterborne | Solvent Base / Waterborne Finish | 22, 23 |
| #### Interior 3-F** ##### Solvent Base Enamel Finish (Gloss as Selected) |  |  |  |
| 1 | Alkali and Moisture Resistant | Alkali and Moisture Resistant |  |
|  | Primer*** | Primer*** |  |
| 2 | Enamel Finish | Enamel Undercoat (tinted) |  |
| 3 | Enamel Finish | Enamel Finish |  |
| 4 |  | Enamel Finish |  |
| #### Interior 3-G ##### Fire Retardant Finish |  |  |  |
|  |  | Follow manufacturer's recommendations. Check local code requirements |  |
| #### Interior 3-H ##### Multi-Colored Coatings |  |  |  |
|  |  | Follow manufacturer's directions for priming and finish coatings on various substrates. |  |

*The Alkyd Primer Sealer is not recommended as a first coat on gypsum drywall.

*Ph reading must be taken so proper primer can be applied. Check manufacturer's recommendation.

*Moisture Content reading must be taken so proper primer can be applied. Check with manufacturer's recommendation.

*Use vapor barrier primer when required.

*Interior 3-E. Often custom grade is sufficient for ceilings; however, premium grade is required on walls.

**Interior 3-F. This system is designed for use on surfaces that possibly will be moist due to condensation.

## New Acoustic Plaster and Tile

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 4-A* ##### Solvent Base Flat Finish |  |  |  |
| 1 | Primer Sealer |  |  |
| 2 | Flat Finish |  |  |
| #### Interior 4-B* ##### Waterborne Flat Finish |  |  |  |
| 1 | Primer Sealer |  |  |
| 2 | Flat Finish |  |  |
| #### Interior 4-C ##### Drywall Finish |  |  |  |
|  | Check with manufacturer's recommendation for coatings systems |  |  |

*Interiors 4-A and 4-B. Should be sprayed with matched color.

## Concrete, Masonry (Except Concrete Masonry Units), Stucco and Stone

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 5-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 5-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 5-C ##### Stipple Finish Solvent Base (Gloss & Texture as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer* | Primer Sealer* |  |
| 2 | Stipple Paint | Enamel Undercoat (tinted) |  |
| 3 |  | Stipple Paint |  |
| #### Interior 5-D ##### Stipple Finish Waterborne (Gloss & Texture as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer* | Primer Sealer* |  |
| 2 | Stipple Paint | Enamel Undercoat (tinted) |  |
| #### Interior 5-E ##### Elastomeric Finish |  |  |  |
|  |  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. Use one manufacturer's complete system. |  |
| #### Interior 5-F ##### Multi-Colored Coating |  |  |  |
|  |  | Follow manufacturer’s directions for priming and finish coatings for the various substrates. Use one manufacturer's complete system. |  |

*Moisture Content reading must be taken so proper coating can be applied. Check with manufacturer's recommendation.

*Use vapor barrier primer when required.

## Concrete Masonry Units (Filled Finish)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 6-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Block Filler | Block Filler |  |
| 2 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 3 | Finish | Finish |  |
| #### Interior 6-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Block Filler | Block Filler |  |
| 2 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 3 | Finish | Finish |  |
| #### Interior 6-C ##### Stipple Finish Solvent Base (Gloss and Texture as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | lvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 2 | Stipple Paint | Stipple Paint |  |
| #### Interior 6-D ##### Stipple Finish Waterborne (Gloss and Texture as Selected) |  |  |  |
| 1 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
|  | Primer Sealer | Primer Sealer |  |
| 2 | Stipple Paint | Stipple Paint |  |

*Moisture Content reading must be taken so proper coating can be applied. Check with manufacturer's recommendation.

*Use vapor barrier primer when required.

## Concrete Floors

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 7-A ##### Painted Finish System Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel Finish (Reduced 25%) | Floor Enamel Finish (Reduced 25%) |  |
| 2 | Floor Enamel Finish | Floor Enamel Finish |  |
| 3 |  | Floor Enamel Finish |  |
| #### Interior 7-B* ##### Enamel Finish System Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Floor Enamel Finish |  |  |
| 2 | Floor Enamel Finish |  |  |
| #### Interior 7-C* ##### Concrete Stain Solvent Base |  |  |  |
| 1 | Stain Finish |  |  |
| 2 | Stain Finish |  |  |
| #### Interior 7-D* ##### Concrete Stain Waterborne |  |  |  |
| 1 | Stain Finish |  |  |
| 2 | Stain Finish |  |  |
| #### Interior 7-E* ##### Zone Marking Solvent Base |  |  |  |
| 1 | Traffic Marking Paint Finish |  |  |
| 2 | Traffic Marking Paint Finish |  |  |
| #### Interior 7-F* ##### Zone Marking Waterborne |  |  |  |
| 1 | Traffic Marking Paint Finish |  |  |
| 2 | Traffic Marking Paint Finish |  |  |

*Only Custom Grade system recommended.

*Moisture Content reading must be taken so proper coating can be applied. Check with manufacturer's recommendation.

*Use vapor barrier primer when required.

## Glass Fiber Reinforced Concrete

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 8-A ##### Paint Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Alkali-Resistant Primer | Alkali-Resistant Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 8-B ##### Paint Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Alkali-Resistant Primer | Alkali-Resistant Primer |  |
| 2 | Finish | Finish |  |
| 3 | Finish | Finish |  |
|  |  | Alkyd Enamel |  |

*Moisture content reading must be taken so proper coating can be applied. Check with manufacturer's recommendation.

*Use vapor barrier primer when required.

## Asphalt, and Asphalt Concrete (Traffic marking only)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 9-A* ##### Zone Marking Solvent Base |  |  |  |
| 1 | Traffic Paint Finish |  |  |
| 2 | Traffic Paint Finish |  |  |
| #### Interior 9-B* ##### Zone Marking Waterborne |  |  |  |
| 1 | Traffic Paint Finish |  |  |
| 2 | Traffic Paint Finish |  |  |

*Only Custom Grade system recommended.

## Cloth Insulation Coverings (Pipes, ductwork, boilers, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 10-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Drywall Joint Compound thinned with Latex Sealer to working consistency | Drywall Joint Compound thinned with Latex Sealer to working consistency |  |
| 2 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
| 3 | Primer Sealer | Primer Sealer |  |
| 4 | Finish | Finish |  |
|  |  | Finish |  |
| #### Interior 10-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Drywall Joint Compound thinned With Latex Sealer to working consistency | Drywall Joint Compound thinned with Latex Sealer to working consistency |  |
| 2 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
| 3 | Primer Sealer | Primer Sealer |  |
| 4 | Finish | Finish |  |
|  |  | Finish |  |
| #### Interior 10-C ##### Aluminum Painted Finish |  |  |  |
| 1 | Drywall Joint Compound thinned with Latex Sealer to working consistency | Drywall Joint Compound thinned with Latex Sealer to working consistency |  |
| 2 | Solvent Base or Waterborne | Solvent Base or Waterborne |  |
| 3 | Primer Sealer | Primer Sealer |  |
| 4 | Aluminum Paint | Aluminum Paint |  |
|  |  | Aluminum Paint |  |

## Exposed Ferrous Metal (Light and Ordinary Exposures) (Except Galvanized)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 11-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Rust-Inhibitive Primer | Rust-Inhibitive Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 11-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Rust-Inhibitive Primer | Rust-Inhibitive Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 11-C ##### Aluminum Paint Finish |  |  |  |
| 1 | Rust-Inhibitive Primer | Rust-Inhibitive Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

## Galvanized (Zinc-coated) Metal

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 12-A ##### Painted Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Galvanized Primer | Galvanized Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 12-B ##### Painted Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Galvanized Primer | Galvanized Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 12-C ##### Galvanized Metal, Aluminum Finish |  |  |  |
| 1 | Galvanized Primer | Galvanized Primer |  |
| 2 | Aluminum Paint | Aluminum Paint |  |
| 3 |  | Aluminum Paint |  |
| #### Interior 12-D ##### Galvanized Metal, High Heat Finish (Color as Selected) |  |  |  |
| 1 | Galvanized Primer | Galvanized Primer |  |
| 2 | High Heat Finish | High Heat Finish |  |
| 3 |  | High Heat Finish |  |

## Steel - High Heat - Interior (Boilers, breeching, furnace fronts, pipe lines, flues, heat exchangers, stacks, turbines)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 13-A ##### Temperature Range up to 1200 degrees F |  |  |  |
|  | Proper coats on bare metal in accordance with manufacturer's directions |  |  |

## Copper, Bronze, Brass

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 14-A ##### Clear Finish |  |  |  |
|  | Consult finish manufacturer for recommendations |  |  |

## Aluminum

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 15-A* ##### Aluminum Finish Solvent Base (Gloss as Selected) |  |  |  |
| 1 | Aluminum Primer | Aluminum Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 15-B* ##### Aluminum Finish Waterborne (Gloss as Selected) |  |  |  |
| 1 | Aluminum Primer | Aluminum Primer |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Only waterborne system recommended.

## Audio / Visual Screen Background

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 16-A ##### Painted Finish Solvent Base |  |  |  |
| 1 |  | Solvent Base / Waterborne Primer |  |
| 2 |  | Finish |  |
| 3 |  | Finish |  |
| #### Interior 16-B ##### Painted Finish Waterborne |  |  |  |
| 1 |  | Solvent Base / Waterborne Primer |  |
| 2 |  | Finish |  |
| 3 |  | Finish |  |

*Must follow premium grade application using a level 5 gypsum drywall finish - see levels of finish.

## Glass (shadowboxes, backdrops, etc.)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 17-A ##### Painted Finish Waterborne (Gloss as selected) |  |  |  |
| 1 |  | Alkyd Primer |  |
| 2 |  | Int/Ext 100% Acrylic Latex |  |
| 3 |  | Int/Ext 100% Acrylic Latex |  |

## Special Coating Systems - Graffiti (Sacrificial & Non-Sacrificial)

| #### FINISH SYSTEM CODE ##### Description |  |  |  |
| --- | --- | --- | --- |
| Coats: | Custom Grade: | Premium Grade: | Product #: |
| #### Interior 18-A* ##### Graffiti Coat Solvent Base |  |  |  |
| 1 | Finish | Finish |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |
| #### Interior 18-A* ##### Graffiti Coat Waterborne |  |  |  |
| 1 | Finish | Finish |  |
| 2 | Finish | Finish |  |
| 3 |  | Finish |  |

*Check Manufacturer's recommendations for coatings system and graffiti removal.

End of Appendix