READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - DESCRIPTION OF WORK

    - The extent and location of “Dock Bumper” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - ASTM 36/A 36M Steel Plates, Shapes, and Bars

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittal shall include the following:

      - Include construction details, material descriptions, dimensions of individual components and profiles, and finishes for loading dock bumpers.

  - SPECIAL WARRANTY

    - Warranty: Sample of special warranty

  - PROJECT CONDITIONS

    - Field Measurements: Verify actual dimensions of construction contiguous with loading dock equipment, including heights of loading docks, by field measurements before fabrication.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MATERIALS

    - Steel Plates, Shapes, and Bars: ASTM 36/A 36M.

  - DOCK BUMPERS

    - Dock bumpers shall be made from resilient, uniform-size, fabric-reinforced rubber pads pressure-laminated between structural steel angles and secured with steel tie rods or bars. Dock bumpers shall be of the size indicated and suitable for mounting directly to a concrete loading dock. The manufacturer shall be W.B. McQuire Co., Pawling Rubber Corp., Schuyler’s Bumpers, Inc., Or Approved Equal.

    - Laminated-Tread Dock Bumper: Fabricated from multiple, uniformly thick plies cut from fabric- reinforced rubber tires. Laminate plies under pressure on not less than two [fill in]-inch- [fill in] diameter, steel supporting rods that are welded at one end to 1/4-inch- (6-mm-) thick, structural-steel end angle and secured with a nut and angle at the other end. Fabricate angles with predrilled anchor holes and sized to provide not less than [fill in] (fill in mm) of tread plies extending beyond the face of closure angles.

      - Thickness: As indicated on Drawings.

      - Horizontal Style: [fill in} inches (fill in mm) high by length indicated on Drawings

  - ATTACHING HARDWARE

    - Attaching hardware shall be recommended by the manufacturer.

    - Anchorage Devices: Hot-dip galvanized-steel anchor bolts, nuts, washers, bolts, sleeves, cast-in-place plates, and other anchorage devices as required to fasten bumpers securely in place and to suit installation type indicated.

- EXECUTION

  - DOCK BUMPERS

    - Attach dock bumpers securely to the concrete loading dock in the indicated locations as recommended by the manufacturer. Make holes in the concrete for anchor bolts by drilling.

- MEASUREMENT AND PAYMENT

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2