READ THIS FIRST

Notice to the Design Engineer, please refer to the Port of Seattle, Facilities and Infrastructure standards for reference before editing this specification.

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Security Systems” Work is shown in the Contract Documents.

    - The Contractor shall provide and install metal conduit as a pathway for the fiber optic cable and the door security cabling as shown on the Drawings. Cabling for door security from the terminal strip at the communication room to the SAC will be provided and installed by others. The Work specified in this section includes two security-related tasks.

      - The Contractor shall provide and install a fiber optic cable connection between selected communications rooms and standalone controllers (SACs). The fiber optic cable will be provided by the Port of Seattle (Port) as described in Section 27 13 00 - Backbone Cabling Requirements. This task includes splicing the fiber optic cable in the selected communications rooms and terminating the cable at a patch panel in the connecting SAC. The Contractor shall provide and install a metal conduit pathway for the fiber optic cable to each SAC.

      - The Contractor shall provide and install security hardware, conduit, and wiring at the communications room doors as shown in the Drawings. The security hardware will be connected to a junction box above each door. The Contractor shall provide and install a conduit pathway and pull cords between the security system junction box above the door in each communications room and a SAC as shown in the Drawings. Copper cabling between the communications room and the SAC will be provided and installed by others.

    - Port-Furnished Materials: The Contractor shall provide all materials except those specifically stated in Division 27 Communications as Port-furnished materials.

  - GOVERNING CODES, STANDARDS AND REFERENCES

    - NEC – National Electrical Code

    - UL – Underwriters Laboratories, Inc.

Insert ICSI or Systemax or NEC and NEMA or IEC standards as applicable to the project.

  - SUBMITTALS

    - Submit materials data in accordance with of Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Product Data: For each type of product indicated. Include data on features, ratings, and performance include.

        - Terminal strips

        - Junction boxes with lockable cover and tamper switch

        - Door security hardware copper cable

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - DOOR SECURITY HARDWARE

    - The Contractor shall provide and install a terminal strip, junction boxes, and connecting conduit for door security hardware as shown on the Drawings. The terminal-strip junction box shall have a hinged, lockable cover and a tamper switch.

    - The Contractor shall provide and install security hardware for the door of each communication room and main distribution room (MDR) as shown on the Drawings. Specifications for door security hardware are provided in Section 08 70 00 Hardware.

    - All material shall be listed and labelled by [U.L.] or [AHJ approved equal].

  - CONDUIT

    - See Section 26 05 33 - Raceways and Boxes, Part 2, for conduit materials.

  - TERMINATION HARDWARE

    - The Contractor shall provide and install termination hardware for the fiber optic cable installed between select communications rooms and SACs. Specifications for termination hardware are provided in Section 27 13 00 - Backbone Cabling Requirements.

    - The Contractor shall provide and install splice trays and execute splicing of the fiber optic cable as described in Section 27 13 00 - Backbone Cabling Requirements and as shown on the Drawings.

- EXECUTION

  - INSTALLATION

    - Door Security Hardware:

      - The Contractor shall connect the door security hardware to the terminal strip and test all connections to ensure that the door security hardware will function properly.

      - Applying hardware-required voltage at the J-box terminal strip per manufacturer's requirements and specifications shall test door security hardware.

    - Fiber Optic Cabling

      - The Contractor shall install Port-provided fiber optic cabling between select communication rooms and adjacent SACs as shown on the Drawings.

    - Conduit

      - The Contractor shall install conduit as indicated in the Drawings.

  - SYSTEM PERFORMANCE

    - System performance requirements shall be the same as those specified in Section 27 13 00 - Backbone Cabling Requirements and Section 27 05 53 - Identification and Labeling.

  - LABELING

    - Labeling requirements shall be the same as those specified in Section 27 13 00 - Backbone Cabling Requirements.

  - POST-INSTALLATION TESTING

    - Post-installation testing requirements shall be the same as those specified in Section 27 13 00 - Backbone Cabling Requirements.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2 and revisions