READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent of “Removal and Alteration” Work includes the removal and disposal of all items and materials, as indicated on drawings and in the specifications. The Contractor shall provide shoring and protection as necessary to prevent damage to existing structures.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - References

      - [Special requirements for Work conducted on, through or across the airfield within the Air Operations Area (AOA) are included in Section 01 35 13.13 - Operational Safety on Airports During Construction].

  - SUBMITTALS

- MATERIALS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - PROJECT INFORMATION

  - PREPARATION FOR MATERIALS

  - FABRICATION, PRODUCTION, & SUPPLY OF MATERIALS

  - MATERIAL REQUIREMENTS

    - Materials For Reuse At The Project Site Area

      - Material indicated on the drawings to be relocated shall be recovered in good condition. Store items in designated locations at the project site in such a manner that the items will not be damaged by the alteration operations.

    - Materials And Equipment To Be Salvaged And Turned Over To The Port

Use one of the following paragraphs, A or B, not both:

      - [List specific items] [List disposition]

OR

      - All material and equipment not reused in the Project and which has any salvage value or possibly can be used at a future date shall be presented to the Port for disposition. If accepted by the Engineer, hold and protect the equipment for moving and subsequent storage by the Port.

      - Contact the Engineer for the delivery point for each salvage item or group of items. All support staging frames shall be provided by the Contractor.

    - Scrap Material

      - Items not specified for reuse and items not salvage to the Port and items not elsewhere specified for specific disposal are designate as scrap material and become the property of the Contractor.

      - Remove scrap material from the Work area immediately. Transport scrap materials. Storage or sale of removed items on the site will not be permitted.

      - See Section 01 74 19 - Construction Waste Management.

  - MATERIAL HANDLING, DELIVERY, & STORAGE

  - DELIVERABLES

  - QUALITY ASSURANCE

- EXECUTION

  - PROJECT INFORMATION

    - Occupancy

      - The Port facilities will be occupied for normal operations during the remodel construction operation. The building remodel shall be phased for construction alteration as indicated on the drawings and in Section 01 11 00 - Summary of Work.

      - Conduct operations in such a manner as to cause the least amount of interference with public traffic and the Port’s operations.

      - The area to be altered will be vacated prior to the scheduled start of the Work in that area.

    - Condition Of Structures

      - The Port assumes no responsibility for the actual condition of the portions of the structures to be removed or altered. The drawings do not show all arrangements and conditions of the structures and appurtenances as they now exist, nor is there any guarantee that fixtures and materials will be in full accord with the drawings. The Contractor shall be responsible for the complete repair of existing construction and equipment affected by the new construction. The Contractor shall inspect the existing construction and the quantity and condition of equipment and materials, and shall assume full responsibility for servicing and repairing of equipment and materials.

      - Repairs to the structure necessitated by the remodeling shall be made using material similar to that of adjacent surfaces. Where finished surfaces, such as paint, tile, wood trim, etc., are disturbed or abraded by remodeling Work, such finished surfaces shall be replaced in kind and color.

    - Alterations Of Existing Construction

      - Dismantling, cutting and removal of a part of the materials or equipment from the existing structures shall be done in such a manner as to prevent unnecessary damage to the remaining structures. Rebuilding or remodeling necessary to produce a completed job necessitated because of removal of the materials or equipment, or because of remodeling of the existing structures, shall become the responsibility of the Contractor, not withstanding that the specific Work may not be shown on the drawings or mentioned in the specifications. The materials and workmanship necessary to accomplish the remodeling Work shall conform to the drawings and to the applicable sections of these specifications.

  - PREPARATION FOR EXECUTION OF WORK

  - EXECUTION OF WORK

    - Traffic

      - Conduct removal operations and the removal of debris to ensure minimum interference with traffic and adjacent occupied or used facilities.

    - Protection

      - Conduct operations to prevent damage by falling debris or other cause to finished area, structures and facilities, as well as persons. Provide shoring, bracing or support to prevent movement or settlement or collapse of portions of the structures to remain.

    - Damages

      - Promptly repair damages caused to adjacent facilities by removal operations as directed by the Engineer and at no additional cost to the Port.

    - Utility Services

      - Do not interrupt existing utilities which service occupied or used facilities, except when authorized by the Engineer. Provide temporary services during interruptions to existing utilities, as acceptable to the Engineer. No less then forty-eight (48) hours’ notice shall be given to the Engineer before any intended interruption of services.

    - Barricades

      - Before starting any cutting or removing any part of the existing building, construct barricades where indicated on the drawings and as required for the protection and routing of public and [Airport]/[Port] personnel through or around the construction area. Construct barricades as stated in Section 01 50 00 - Temporary Facilities and Controls. Protect all finished areas enclosed within barricades which are not included in the demolition Work. Where necessary, provide suitable hinged access doors, with latch, in barriers. Remove the barriers upon completion of the contract.

    - Removal

      - Removal Operations: Use such methods as required to complete the Work within the limitations of governing regulations and other special requirements as specified.

      - Proceed with removal operations in a systematic manner.

      - Equipment used near all areas of occupancy, including public spaces, shall be of the lowest noise intensity possible.

      - Locate removal equipment and remove materials so as not to impose excessive loads to supporting walls, floors, or framing.

    - Disposal Of Materials

      - Remove from the site all debris, rubbish and other materials resulting from removal operations. Keep public areas clean at all times.

      - Burning of removed materials from removal operations will not be permitted on Port property.

      - Legally dispose of off Port property all scrap and waste materials, and protect the Port from all damages that may arise there from.

      - See Section 01 74 19 - Construction Waste Management.

  - DELIVERABLES

  - QUALITY ASSURANCE

- MEASUREMENT AND PAYMENT

  - MEASUREMENT

    - “Removal and alterations” shall be measured as a unit and paid for at the contract lump sum price stated in the schedule of unit prices. The price shall be full compensation for furnishing all labor, equipment and materials necessary for removal and alteration Work, including disposal, as indicated on the drawings.

  - PAYMENT

    - Removal and Alterations – paid by “lump sum”

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2