READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Portable Fire Extinguishers and Cabinets” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - Underwriters Laboratories fire extinguisher

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Manufacturer’s Data: Submit the manufacturer’s color samples

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - FIRE EXTINGUISHERS

    - Provide fire extinguishers for each extinguisher cabinet and other locations as shown on the drawings. Furnish only new fire extinguishers which are approved and labeled by Underwriters Laboratories.

      - Fill and service extinguishers in accordance with [Port of Seattle Fire Department] [City of Seattle Fire Department].

      - Provide the manufacturer’s standard mounting brackets for extinguishers not located in cabinets.

    - Water Pressurized (2A): 2-1/2 gallon size, stainless steel container with pressure-indicating gauge, for Class A fires.

      - Dry Chemical (4BC): 2-1/2 pound, enameled steel container with pressure-indicating gauge, for Class B and Class C fires. [Review with [Port of Seattle Fire Department] [City of Seattle Fire Department]].

  - FIRE EXTINGUISHER CABINETS

    - General: Provide fire extinguisher cabinets suitable for housing one standard-size fire extinguisher as specified, as follows: [Review with [Port of Seattle Fire Department] [City of Seattle Fire Department]].

    - Recessed: Trim with beveled or rounded edges in contact with wall surface. Provide square corners with approximately 1-1/4 inch-wide trim, unless otherwise shown.

    - Metal Gauge: Provide cabinets fabricated of the following minimum equivalent steel gauges:

      - Box: 20 gauge

      - Trim Frame: 18 gauge

      - Tubular Door Perimeter Frame: 20 gauge

    - Construction: One-piece tubular door frames, mitered and welded. One-piece metal trim frame, to suit cabinet style required. Weld all joints and grind smooth Provide the manufacturer’s standard steel box with white, baked enamel interior finish and primed exterior finish.

    - Doors: Break-glass panel, DSA glass, with inside latch and lock.

    - Door Hardware: Continuous-type hinge permitting door to open 180 degrees. Provide either lever handle with cam action latch, or door pull and friction latch.

- EXECUTION

  - INSTALLATION

    - Install in a location and at a mounting height to comply with the requirements of the [Port of Seattle Fire Department] [City of Seattle Fire Department]. Prepare recesses in walls as required. Securely fasten to structure, square and plumb, in accordance with the manufacturer’s instructions.

    - Wherever the exact location of surface-mounted units is not shown, locate as directed by the Engineer.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2