READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Preformed Metal Fascia” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - ASTM A653 / A653M – 11; Standard Specification for Steel Sheet, Zinc-Coated (Galvanized) or Zinc-Iron Alloy-Coated (Galvannealed) by the Hot-Dip Process

  - SUBMITTALS

    - Submit materials data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Submit certified laboratory test reports for required performance tests.

      - Submit the manufacturer’s standard warranty on factory-applied finish of performed metal fascia.

      - Samples: Submit three, two foot long, full-width samples of the type of preformed metal fascia panels including bent panels or special shapes, complete with factory-applied finish in color selected by the Engineer. Samples will be reviewed by the Engineer for pattern, texture and color only. Compliance with other properties is a requirement of the Contract.

      - Shop Drawings: Submit shop drawings showing the profiles of preformed metal fascia panel units and the details of forming, jointing, internal supports, anchorages, trim, flashing and accessories in accordance with Section 01 33 00 - Submittals. Show details of weatherproofing at edges, terminations and penetrations of the preformed metal fascia panel Work. Show small-scale layout and elevations of the entire Work.

      - Close out documents

  - QUALITY ASSURANCE

    - Subcontractor: The erection of the preformed metal fascia may be subcontracted to either the manufacturer or an experienced erection firm licensed by or acceptable to the manufacturer.

    - Performance Test Standards: Provide preformed metal fascia systems which have been pre-tested and certified by the manufacturer under installed conditions as indicated for resistance to air and water infiltration and structural deflection and failure; ASTM [ ].

    - Field Measurements: Wherever possible, take field measurements prior to completion of shop fabrication and finishing of preformed metal fascia. Do not delay job progress; allow for trimming where final dimensions cannot be established before fabrication.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MANUFACTURER

Competition waiver to be submitted to CPO for ASC Building Products to be the only approved product for the Port.

    - ASC Building Products, No Equal.

  - PANEL MATERIALS

Competition waiver to be submitted to CPO for Cal-Clad panels to be the only approved product for the Port.

    - Provide Cal-Clad panels, No Equal, with 36-inch coverage and ribs at 12 inches on center, as detailed. The details shown are based on standard details by one or more manufacturers. It is intended that similar details by other manufacturers will be acceptable, provided that they comply with the size requirements and profile requirements specified.

    - Provide fascia panels and other exposed accessories and trim formed from steel sheets conforming to ASTM [ ] Grade A, 26 gauge.

  - MISCELLANEOUS MATERIALS

    - Refer to other specification sections for structural support systems or substrates shown to receive preformed metal fascia panels.

    - Assembly and Installation Accessories: Provide the manufacturer’s standard fasteners, brackets, clips, anchoring devices, spacers, flashings, closures, adhesives, joint sealers and other components needed for a complete permanently weatherproof installation. Use materials which are non-corrosive, non-deteriorating and compatible with the panel faces.

    - Trim: Provide trim components indicated, if any, as part of the preformed metal fascia panel Work.

  - PANEL FABRICATION

    - General: Comply with the dimensions, profile limitations, gages and fabrication details; and, to the extent not detailed, provide the manufacturer’s standard product fabrication of panels.

    - Performance: Fabricate components and assemble units to comply with the performance requirements specified for the completed installation of the Work.

  - METAL PANEL COATINGS

    - General: Pre-finish exposed metal sheets to the greatest extent possible prior to forming and panel fabrication.

    - Panels shall be coated with “Zincalume,” a zinc/aluminum alloy, applied to the sheet steel by continuous hot-dip process to a nominal thickness of 1.9-mils.

    - The exterior finish shall include a 0.2-mil, baked-on, epoxy base primer and a 0.8-mil, silicone, modified-polyester finish coat for a total minimal dry film thickness of 1.0-mil. The finish coat color shall be [chestnut brown, 0021-05], [ ].

    - Concealed Steel Coating: On concealed faces of steel panels, provide the panel manufacturer’s standard rust-inhibitive coating. After proper cleaning, pretreatment and conversion coating, apply one coat of rust-inhibitive metal primer and one coat of metal enamel to 1.0-mils of dry film thickness.

- EXECUTION

  - INSTALLATION

    - General: Comply with the panel manufacturer’s instructions for assembly, installation and erection of preformed metal fascia.

    - Anchor component parts of the preformed metal fascia panels securely in place, providing for necessary structural movement.

    - Tolerance: Erect the Work plumb, level and true to line with tolerances not exceeding 1/4 inch in runs of 20’-0”.

    - Fasteners: Per instructions and recommendations of preformed metal fascia panel manufacturer.

    - Sealants or Gaskets: Install sealants or gaskets for preformed metal fascia panel Work as indicated and as required for performance. Gaskets, if used, shall be as recommended by the preformed metal fascia panel manufacturer.

    - Damaged Material: Remove and replace panels and component parts of the Work (including finish) which have been damaged beyond successful repair, as directed by the Engineer. Repair minor damage.

  - CLEANING AND PROTECTION

    - Clean exposed surfaces of preformed metal fascia panel Work promptly after completion of installation. Comply with the recommendations of both the panel manufacturer and coating manufacturer.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2