READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Preformed Metal Sandwich Panel” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - NAAMM Standard Test TM-1 static and dynamic methods

    - Type 3004-H26 aluminum facing sheet

  - SUBMITTALS

    - Submit materials data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Submit certified laboratory test reports for required performance tests.

      - Submit the manufacturer’s standard warranty on factory-applied finish of preformed metal siding.

      - Samples: Submit three, two-foot long, full width samples of the type of preformed metal sandwich panels, including bent panels or special shapes, complete with factory-applied finish in color selected by the Engineer. Samples will be reviewed by the Engineer for pattern, texture and color only. Compliance with other properties is a requirement of the Contract.

      - Shop Drawings: Submit shop drawings showing the profiles of preformed metal sandwich panel units and the details of forming, jointing, internal supports, anchorages, trim flashing and accessories in accordance with Section 01 33 00 - Submittals. Show details of weatherproofing at edges, terminations and penetrations of the preformed metal sandwich panel Work. Show small-scale layout and elevations of entire Work.

  - QUALITY ASSURANCE

    - Subcontractor: The erection of the preformed metal sandwich panels may be subcontracted to either the sandwich panel manufacturer or an experienced erection firm licensed by or acceptable to the preformed metal sandwich panel manufacturer.

    - Performance Test Standards: Provide preformed sandwich panel systems which have been pre-tested and certified by the manufacturer under installed conditions as indicated for resistance to air and water infiltration and structural deflection and failure; NAAMM Standard Test TM-1; static and dynamic methods.

    - Field Measurements: Wherever possible, take field measurements prior to completion of shop fabrication and finishing of preformed metal siding. Do not delay job progress; allow for trimming where final dimensions cannot be established before fabrication.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - PANEL MATERIALS

    - Provide the sizes and thickness of panels as shown on the drawings, with the edge condition and profile as detailed. The details shown are based upon standard details of one or more manufacturers. It is intended that similar details by other manufacturers will be acceptable, provided they comply with size and profile requirements.

    - Provide aluminum alloy and temper for exposed sheets as recommended by the aluminum manufacturer and processor to comply with the requirements of performance, fabrication, application of finish and color.

    - Laminate aluminum sheets to expanded polystyrene core, self-extinguishing foam plastic. Nominal density of two pounds per cubic foot, two inches thick, with waterproof adhesive selected for durability and extreme temperature conditions, achieving a flatness tolerance (bow at any point, any direction) of not more than 1/16 inch in a two foot run and not more than 1/8 inch for an entire panel. Provide all reinforcing channels, angles, required to maintain the flatness specified.

    - Provide aluminum facing sheet of sheet alloy Type 3004-H26, plain, not less than 0.04-inch thick.

    - Provide aluminum back sheet of 0.032-inch aluminum.

  - MISCELLANEOUS MATERIALS

    - Panel Internal Framing: Provide the panel manufacturer’s standard metal subgirts and other structural or spacing members designed for required structural anchorage and loading capacity and selected for compatibility with panel faces.

    - Refer to other sections of these specifications for structural support systems or substrates shown to receive preformed metal sandwich panels.

    - Integral Panel Subsupport System: Provide panel support systems where shown as an integral part of the preformed metal sandwich panel system. Provide the panel manufacturer’s standard framing designed for 20 psf loading, unless otherwise indicated, of metal members selected for compatibility with panels and properly coated or otherwise protected against deterioration for the life of the building.

    - Assembly and Installation Accessories: Provide the manufacturer’s standard fasteners, brackets, clips, anchoring devices, furring strips, spacers, flashings, closures, adhesives, joint sealers and other components needed for a complete, permanently weatherproof installation. Use materials which are non-corrosive, non-deteriorating and compatible with the panel faces.

    - Trim: Provide trim components indicated, if any, as part of the preformed metal sandwich panel Work. Except as otherwise indicated, match the material and finish of the exterior panels.

  - PANEL FABRICATION

    - General: Comply with the dimensions, profile limitations, gauges and fabrication details shown; and, to the extent not shown, provide the manufacturer’s standard product fabrication in full height panels.

    - Performance: Fabricate components and assemble units to comply with the performance requirements specified for the completed installation of the Work.

  - PAINT FINISH

    - Finishing: After the aluminum has been cleaned by an etching process, apply a baked synthetic resin enamel. Color shall match existing panels to the satisfaction of the Engineer.

- EXECUTION

  - INSTALLATION

    - General: Comply with the panel manufacturer’s instructions for assembly, installation and erection of preformed metal siding.

    - Anchor component parts of the preformed metal sandwich panels securely in place, providing for necessary structural movement.

    - Tolerances: Erect the Work plumb, level and true to line with tolerances not exceeding 1/4 inch in runs of 20’-0”.

    - Fasteners: Provide a concealed-fastener installation system with no fasteners exposed on the exterior faces of the Work.

    - Sealants or Gaskets: Install sealants or gaskets for the preformed metal sandwich panel Work as indicated and as required for performance. Gaskets, if used, shall be as recommended by the preformed metal sandwich panel manufacturer.

      - Refer to Section 07 92 00 - Joint Sealers for sealants between the Work and adjoining Work.

    - Damaged Material: Remove and replace panels and component parts of the Work (including finish) which have been damaged beyond successful repair, as directed by the Engineer. Repair minor damage.

  - CLEANING AND PROTECTION

    - Clean exposed surfaces (exterior and interior) of preformed metal sandwich panel Work promptly after completion of installation and remove strippable coating (if any). Comply with the recommendations of both the panel manufacturer and the coating manufacturer.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2