READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Resilient Flooring” Work is indicated in the Contract Documents. Work includes providing base, special moldings, underlayment and all associated materials.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - ASTM-F1700 vinyl tile

    - ASTM F1861 base

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Submit samples of each type of flooring material specified, including accessories, for approval of color, texture and pattern.

      - Submit layout drawings showing location of all panels.

      - Employ at least one person who shall be present at all times during the execution of this portion of the Work and who shall be thoroughly trained and experienced in the installation of the various components specified and who shall direct all Work performed under this section.

  - PRODUCT HANDLING

    - Delivery and Storage of Materials: Deliver all materials to the job site in unopened containers with manufacturers’ labels thereon; do not open containers or remove labels until the Engineer inspects and approves.

    - Precautions: Maintain the temperature of resilient floor covering materials at not less than 70°F for 48 hours just prior to laying.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - VINYL TILE

    - ASTM-F1700, Class I, Type [A] [B], as manufactured by Amtico, Armstrong, Azrock, Roppe, Or Approved Equal. Standard vinyl flooring tiles; 12-inch x 12-inch size; 3/32-inch thickness.

  - BASE

    - ASTM F1861, Type TP, rubber base, four inch high, top-set, 1/8-inch thickness, with molded corners, as manufactured by Burke, Roppe, Mercer, Or Approved Equal. Color shall be as selected by the Engineer.

  - OTHER MATERIALS

    - General: Provide accessory materials, equipments, tools and methods as required for a complete installation.

    - Primers, Adhesives and Sealers: Water-resistant types made or recommended by covering manufacturer. Submit a list of types proposed and obtain approval before proceeding.

    - Reducers: Johnson, Mercer, Or Approved Equal. Standard vinyl floor reducer, thickness to suit adjacent covering thickness; one inch width. Match adjacent flooring material.

- EXECUTION

  - INSTALLATION

    - General:

      - Install all materials after concrete and masonry is completely dry, in strict accordance with the manufacturer’s recommendation and approved by the Engineer; install floor covering by cementing directly to substrate.

      - Condition of Surfaces: Verify the condition of substrates prior to installation; installation of coverings will be construed as acceptance of all conditions of the installation, including temperatures, moisture contents, cleanliness and general condition of substrate. Should surfaces be deemed unacceptable for the installation of covering, notify and await the directions of the Engineer. When required, at no additional cost to the Port, install underlayment material as recommended by the manufacturer.

    - Installation of Vinyl Tile: Shuffle tile so that finished floors are free of abrupt changes in color or texture. Apply adhesive recommended by the manufacturer for the conditions of the application, spreading adhesive evenly over the surface. Lay tile with the pattern or grain of the tile running in alternate direction; cut and fit tile to room shapes and profiles as scheduled. Lay tile square with the room or space axis.

    - Installation of Stair Covering: Install materials in accurate lengths as required; adhere to substrate.

    - Installation of Base: Upon unrolling of the base material, lie out and cut to as long lengths as practicable; carefully match edges at all seams, or double-cut adjoining lengths. Arrange with the joints so as to be practically invisible. Adhere base materials securely to substrate, laying the material in straight lines with top edges level and true. Form internal and external corners.

    - Installation of Accessories:

      - Install the specified accessories in accurate locations as indicated or scheduled on the approved drawings; firmly bond to substrate backing.

      - Provide reducers wherever covering terminates and exposes the edge of floor covering material; where floor covering terminates at a door opening, center stripes under the door. Adhere vinyl reducers securely to floor surfaces in straight, true lines.

  - FINISHING

    - After floor covering, base and accessories have set sufficiently, wash with neutral cleaner; apply two coats of non-slip wax, type recommended by the floor covering materials manufacturer. Buff by machine to a smooth, dull-gloss finish; hand-buff areas inaccessible to machine. Leave surfaces clean and smooth, free from waves, buckles, cracks, projecting edges and other defects.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2