READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Acoustical Wall System” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - ASTM C423

    - ASTM E84 surface burning characteristics

      - Smoke developed: 5 or less

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittal shall include the following:

      - Provide 2 ft. x 2 ft. square sample of the acoustical panel system with the custom tucked upholstery pattern as shown on the drawings and with representative attachment hardware.

      - Submit layout drawings showing location of all panels.

  - QUALITY ASSURANCE

    - Provide a demountable acoustical wall panel system with surface burning characteristics as indicated below, which have been determined by testing in accordance with ASTM E84 by a testing organization acceptable to the authorities having jurisdiction.

      - Flame spread: 25 or less

      - Fuel contributed: 20 or less

  - PRODUCT HANDLING

    - Protect acoustical units from excessive moisture in shipment, storage and handling. Deliver in unopened bundles and store in a dry place with adequate ventilation and air circulation to prevent damage due to moisture build-up. Do not deliver to site until wet Work such as drywall taping and the like have been completed and normal ambient moisture conditions have been restored.

  - PRODUCT CONDITIONS

    - Do not begin installation until spaces to receive acoustical wall panels have been enclosed and maintained at approximately the same conditions of temperature and humidity as planned for occupancy.

    - Maintain the conditions of temperature and humidity that are recommended by the manufacturer of the acoustical panels.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MATERIALS

    - Back Mounted, Edge Reinforced, Wall Panels: Manufacturer’s standard panel construction consisting of fabric facing laminated to edge reinforced molded glass fiber core on front edges and back border and complying with the following requirements:

      - Fabric facing: Material, color, pattern and texture as selected by the Engineer from the manufacturer’s standard range of fabrics and fabricated into the custom tucked facing pattern as indicated on the drawings.

      - Thickness: 1-3/16 inch

      - NRC: Not less than 0.80 per ASTM C423, A (#4 mounting)

      - Edge detail: Square

      - Panel size: As noted on the drawings

    - Subject to compliance with requirements provide products manufactured by one of the following:

      - Armstrong World, Inc.

      - Decoustics

      - Capaul Div., Acoustiflex Corp.

      - Or Approved Equal.

    - Back Mounting Accessories: Mechanical fasteners including metal clips, hangers, concealed inside trim and other attachment components as required by manufacturer for fully concealed fastening to walls of back mounted panels.

  - FABRICATION

    - Attach facing material to core so that there are no wrinkles, sags or blisters.

    - Align fabric texture parallel to edges and ends.

    - Fabricate outside corners with fully concealed internal metal reinforcement connecting short panel segments with continuous facing fabric extending around and a short distance on either side of outside corners to a butt joint as shown on the drawings to avoid butt to back joint conditions and outside trim at corners.

    - Prefabricate panels with openings for wall items to avoid field cutting of panels to the greatest extent possible.

- EXECUTION

  - INSTALLATION

    - Install acoustical wall panel system plumb, with butt joints fitted tight, snug to adjacent Work, in proper alignment as noted on the drawings and in strict accordance with manufacturer’s instructions.

    - Arrange units symmetrically on wall unless noted otherwise on the layout drawing.

    - Cut units to be at least 50 percent of unit width, with facing extended over cut edge to match uncut edge. Scribe acoustical wall units to fit adjacent Work. Locate cut edges in inside corners.

    - Apply accessories and fastenings as recommended by manufacturer.

  - EXTRA STOCK

    - Spare and extra parts shall be identified for all products, but not provided. Include spare parts information in Operation and Maintenance Manuals.

Include spare parts information in Operation and Maintenance Manuals.

Deliver stock of maintenance material to the Port.

Furnish maintenance material which matches installed products, packaged with protective covering for storage and identified with appropriate labels.

Provide sufficient material and accessories for the fabrication and attachment of one complete full sized panel.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2