READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Wall Covering” Work is indicated in Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - [1. 09720 2.01.B. Seattle Fire Department flame spread]

    - [Port of Seattle Fire Department flame spread.]

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Provide MSDS for all materials

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver sealers, adhesive and accessories specified herein to the job site in sealed labeled containers each bearing the manufacturer’s labels.

    - Store wall covering materials laid flat, not upright, in a dry area with heat maintained at 45°F minimum or higher when recommended by wall covering manufacturer with humidity normal to the space or area where the wall covering is to be installed.

    - Take all necessary measures to eliminate fire hazards and prevent spontaneous combustion of solvents and solvent-based materials used and conform to the requirements of the applicable regulatory agencies.

    - Where toxic materials, such as toxic and explosive solvents, are used, provide appropriate safeguards against inherent hazards. Conform to the manufacturer’s recommendations and to the requirements of the applicable regulatory agencies.

  - QUALITY ASSURANCE

    - Standards: Conform with manufacturer’s printed instructions, or as specified herein, with the most stringent method and practice to govern. Firmly adhering fabric wall covering in place for long life under hard use, insuring a high quality installation.

    - Codes: Conform to all pertinent codes and regulations.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver sealers, adhesive and accessories specified herein to the job site in sealed labeled containers each bearing the manufacturer’s labels.

    - Store wall covering materials laid flat, not upright, in a dry area with heat maintained at 45°F minimum or higher when recommended by wall covering manufacturer with humidity normal to the space or area where the wall covering is to be installed.

    - Take all necessary measures to eliminate fire hazards and prevent spontaneous combustion of solvents and solvent-based materials used and conform to the requirements of the applicable regulatory agencies.

    - Where toxic materials, such as toxic and explosive solvents, are used, provide appropriate safeguards against inherent hazards. Conform to the manufacturer’s recommendations and to the requirements of the applicable regulatory agencies.

  - ENVIRONMENTAL CONDITIONS

    - Temperature:

      - Do no wall covering Work until surfaces and materials have been maintained at 60°F minimum, or higher if recommended by wall covering manufacturer, for three days before Work begins.

      - Maintain minimum or above temperatures during the entire installing time and until adhesives have fully dried or cured.

    - Lighting: Maintain minimum lighting level of 15 candlepower per square foot on the surfaces to receive wall covering, using temporary lighting, if necessary, to attain specified level.

    - Ventilation: Provide adequate continuous ventilation as required for the scheduled wall coverings and adhesives required during the Work and until adhesives have fully dried or cured. There shall be no Work involving open flame on spark generation during wall covering installation.

  - QUANTITIES

    - Spare and extra parts shall be identified for all products, but not provided. Include spare parts information in Operation and Maintenance Manuals.

Include spare parts information in Operation and Maintenance Manuals.

For future maintenance, five yards of fabric wall covering shall be provided to the Port of Seattle by the Contractor in excess of that required.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - FABRIC AND VINYL WALL COVERING

    - Refer to [Finish Furnishing and Equipment Schedule] by Engineer for fabric wall covering type, and color, texture, and pattern.

    - All material shall conform to applicable codes with regard to flame spread. Contractor shall submit sample of material and certification from independent testing lab to the [Port of Seattle] [Seattle Fire Department] [Port of Seattle Fire Department].

    - As part of this Contract, wall covering shall be sent to Kiesling Hess for Class A flame-retardant treatment prior to installation.

  - LINING PAPER

    - Install generally under Shiki, cork, burlap, grass cloth, foil, flock on foil, or where existing conditions require lining paper as recommended by wall covering manufacturer.

  - PRIMER AND ADHESIVE

    - Primer and adhesive shall be of a type recommended by the wall covering manufacturer and shall contain a non-mercury based mildewcide. Adhesive shall be strippable type. Adhesive to install cap shall be of a type recommended by the manufacturer of the wainscot cap. .

- EXECUTION

  - CONDITION OF SURFACES

    - Inspect all surfaces to receive wall covering to verify that they are in proper condition for installation.

    - Notify the Engineer in writing of all conditions including, but not limited to, cracks, voids, ridges, oils, grease, moisture, porosity, indelible and water soluble crayon, ball or felt tip pen marked, and foreign materials and substances which will prevent satisfactory finished Work; do no Work until said conditions have been corrected.

    - Do not start Work until other Work requiring penetration of wall covering has been completed.

    - The installation of wall covering shall be an indication of acceptance of the substrate and the fabric wall covering installer automatically assumes the responsibility for any unacceptable finished Work caused by substrate conditions.

  - PROTECTION

    - Provide sufficient drop cloths, shields and protective equipment to prevent primers or adhesives or wall covering from fouling adjacent surfaces and in particular in storage and preparation areas.

    - Place waste, cloths and materials which may constitute a fire hazard in closed metal containers and remove from site daily.

    - Coordinate Work with other trades where practicable; remove electrical outlet and switch plates, mechanical diffusers, escutcheons, registers, surface hardware, fittings and fastenings prior to starting Work; store during and replace after Work.

  - APPLICATION OF ADHESIVES

    - Use only adhesive specified.

    - Mix and apply adhesives by methods and means strictly in accordance with the manufacturer’s directions for the type of material being applied to assure firm bonding of wall covering to substrate.

    - Take special care to prevent adhesive from getting on the surface of wall covering.

  - LINING PAPER INSTALLATION

    - Adhesively apply lining paper under wall coverings where scheduled, recommended by the manufacturer of the wall covering or in accordance with good trade practice.

  - WALL COVERING INSTALLATION

    - Verify the type, pattern, color and quantity of wall covering for each location scheduled to receive wall covering.

    - Installed material shall be secure, smooth, clean, without wrinkles, gaps or overlaps.

    - Wall covering is to extend continuously behind items which are close to but not bolted to or touching walls.

    - Install materials explicitly following the wall covering manufacturer’s recommendations.

  - SALVAGE

    - Remove salvage with a sharp blade using a metal straight edge as a guide.

    - Trim additional salvage where required to achieve a color and pattern match at seams.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2