READ THIS FIRST

Notice to the Design Engineer, please refer to the Port of Seattle, Facilities and Infrastructure standards for reference before editing this specification.

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Toilet Accessories” Work is shown in the Contract Documents.

    - Types: Toilet accessories required include the following:

      - Paper towel dispensers/disposals

      - Hand dryers

      - Soap dispensers

      - Toilet tissue dispensers

      - Toilet seat cover dispensers

      - Recessed Sanitary napkin dispensers

      - Sanitary napkin disposal units

      - Mirrors

      - Waste Paper Basket

      - Wardrobe shelves

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - AISI Type 302/304.

    - ASTM B 456, Type SC2

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittal shall include the following:

      - Setting Drawings: Provide setting drawings, templates, instructions, and directions for installation of anchorage devices and cut-out requirements in other Work.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MANUFACTURERS

    - Manufacturers' names and catalog numbers listed establish the standard of quality required but are not restrictive.

    - Similar and equal products of other manufacturers may be submitted for approval of the Engineer under the provisions of Section 01 25 00 - Substitutions.

    - Approved manufacturers are American Specialties Inc., Bobrick Washroom Equipment Inc., Bradley Washroom Accessories, McKinney/Parker, Or Approved Equal.

  - MATERIALS, GENERAL

    - Stainless Steel: AISI Type 302/304, with polished No. 4 finish, 22-gauge (0.034 inch) minimum, unless otherwise indicated.

    - Fasteners: Screws, bolts, and other devices of same material as accessory unit or of galvanized steel where concealed.

    - Chromium Plating: Nickel and chromium electro-deposited on base metal, ASTM B 456, Type SC2

  - TOWEL CABINETS

    - Towel cabinets shall be “Reserv-A-Roll” No. 800, Bobrick B-3944 (Note to Designer - Describe salient characteristics), Or Approved Equal.

  - TOILET PAPER HOLDERS

    - Toilet paper holders shall be twin-dispenser type, dull chrome finish, Marathon, (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - SOAP DISPENSERS

    - Soap dispensers shall be Bobrick B-865 (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - SANITARY NAPKIN DISPENSER

    - Sanitary napkin dispenser shall be West Chemical Company Kotex machine (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - SANITARY NAPKIN RECEPTACLE

    - Sanitary napkin receptacle shall be Mipro No. 2811 Kotex receptacle (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - WASTE PAPER BASKET

    - Waste paper baskets shall be Bobrick B-269 (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - TOILET SEAT COVER DISPENSER

    - Dispenser shall be Bobrick B-304 (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - SOAP TANK

    - Tank shall be Bobrick B-930 (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

  - MIRRORS

    - Stainless steel angle frame, 3/4 x 3/4-inch heavy gauge, satin finish. Galvanized steel back equipped with concealed locking set screws for securing mirror to wall hanger. Bobrick Series B-290 (without shelf) (Note to Designer - Describe salient characteristics) (Note to Designer – Provide 2 manufacturers), Or Approved Equal.

    - Vandal resistant anti-graffiti safety film minimum 4 mil clear: Armorcoat Safety & Security film, Solargard window film, 3M Safety & Security film or approved equal.

  - WARDROBE SHELVES

- EXECUTION

  - GENERAL

    - Provide and coordinate with all other trades adequate backing, blocking, or bracing to install the products level and plumb. Furnish inserts and anchoring devices which must be set in concrete or built into masonry; coordinate delivery with other Work to avoid delay.

    - Surface-Mounted Toilet Accessories, General: Except where otherwise indicated, fabricate units with tight seams and joints, exposed edges rolled. Hang doors or access panels with continuous stainless steel piano hinge. Provide concealed anchorage wherever possible.

    - Only unobtrusive stamped logo of manufacturer, as approved by the Engineer, is permitted on exposed face of toilet or bath accessory units. On either interior surface not exposed to view or back surface, provide additional identification by means of either printed, waterproof label, or stamped name plate, indicating manufacturer's name and product model number.

  - INSTALLATION

    - Install one seat cover dispenser in each toilet stall.

    - Install towel cabinets and disposal receptacles, toilet paper holders, soap dispensers, sanitary napkin dispensers and receptacles, and a unitized wardrobe shelf according to the manufacturer’s directions at the location indicated on the drawings.

  - ADJUSTING AND CLEANING

    - Adjust toilet accessories for proper operation and verify mechanisms function smoothly. Replace damaged or defective items. Clean and polish exposed surfaces after removing temporary labels and protective coatings.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2

08/6/2019 Added vandal resistant anti-graffiti safety film to mirrors in 2.11