READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of “Loading Dock Equipment” Work including manual dock levelers, dock bumpers and truck shelters as indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - None.

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittal shall include the following:

      - Shop Drawings: For loading dock equipment. Include plans, elevations, sections, details, and attachments to other Work.

      - Detail equipment assemblies and indicate dimensions, weights, loads, required clearances, method of field assembly, components, and location and size of each field connection.

  - QUALITY ASSURANCE

    - Equipment shall be installed by personnel who are thoroughly trained and experienced in the installation of the specified equipment. The final installation shall be approved by the authorized distributor prior to final inspection.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - DOCK BUMPERS

    - Dock bumpers shall be made from resilient, uniform-size, fabric-reinforced rubber pads pressure-laminated between structural steel angles and secured with steel tie rods or bars. Dock bumpers shall be of the size indicated and suitable for mounting directly to a concrete loading dock. Acceptable manufacturers are W.B. McQuire Co., Pawling Rubber Corp., Schuyler’s Bumpers, Inc., Or Approved Equal.

  - RIGID FRAME TRUCK SHELTERS

    - Rigid frame truck shelters shall meet the following requirements:

Competition waiver to be submitted to CPO for Kelley Tuf-Seal to be the only approved product for the Port.

      - Kelley “Tuf-Seal” Model TS-1006, rigid frame truck shelters, No Equal, for 10’-0” wide x 10’-0” high doors with 3’-10” height dock.

      - Frame members are factory painted wood, kiln dried, select, complete with translucent fiberglass top and side panels.

      - Base fabric shall be 40 oz. hypalon brown.

      - Covers shall have built in structural grade thermo set fiberglass stays in a “Z” pattern at the top plus five equally spaced horizontally. 14 inches on center.

      - Rubber wind straps between head curtain and side frame.

      - Wear pleats on head curtain (8) 40 oz. hypalon brown.

      - Bottom wear panels on side curtains 19 inches high three layers.

      - Two wedge shaped foam bottom seals tapered from eight inches to two inches in thickness.

      - Aluminum trim angle.

      - Heavy duty structural steel bumper guards six inches channel.

  - MECHANICAL DOCK LEVELERS

Competition waiver to be submitted to CPO for Kelley to be the only approved product for the Port.

    - Kelley Model No. M708W, 27,000 lbs. capacity, 7’-0” wide x 8’-0” long with 16 inch extended lip, walk down type, yieldable, No Equal.

    - Work range: 24 inches to 12 inches above and 12 inches below dock height.

    - Below dock controls required.

    - Full range toe guards required.

    - Safety cross - traffic legs required.

  - ATTACHING HARDWARE

    - Attaching hardware shall be recommended by the manufacturer.

- EXECUTION

  - PREPARATION

    - Examine all surfaces to receive the equipment specified herein. Verify all dimensions of in-place and subsequent construction affecting this equipment installation. Installation of any equipment constitutes acceptance of all adjacent construction.

  - INSTALLATION

    - Attach all equipment securely to the concrete loading dock in the locations to be indicated on final construction documents as recommended by the manufacturer. Provide holes in the concrete for anchor bolts by drilling, or by block-outs for required inserts when placing concrete.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2