READ THIS FIRST

This Project Spec Document may need additional modifications to suit your project. It is recommended that you proofread each section, paying attention to any “Notes” boxes such as this one--you should remove these “Notes” sections as you go. Also, do a search for all bracket characters “ [ ] “ as they are used to show you areas containing options or project specific details (you can use Microsoft Word’s Find feature {Ctrl-F} to jump to an open bracket “ [ “ character quickly). Again, these bracket characters should be removed.

It is important that every paragraph be numbered to allow for easy referencing. If you use the document’s built in styles and formatting your outline should be fine (turn on the formatting toolbar by going to View > Toolbars > Formatting). Most paragraphs will use the style “Numbered Material” and can be promoted (Shift) or demoted (Shift-Tab).

You should not have to manually enter extra spaces, carriage returns or outline characters such as A, B, C, or 1.01, 1.02; the formatting will do this for you. The entire document is 11 pt. Arial. If you paste items in, you may need to reapply the “Numbered Material” format.

- GENERAL

  - SUMMARY OF WORK

    - The extent and location of the “Plastic-Laminate-Faced Access Panels” Work is indicated in the Contract Documents.

  - GOVERNING CODES, STANDARDS, AND REFERENCES

    - None.

  - SUBMITTALS

    - Submit material data in accordance with Section 01 33 00 - Submittals. Furnish manufacturers’ technical literature, standard details, product specifications, and installation instructions for all products.

    - Submittals shall include the following:

      - Manufacturer’s Data: Submit the manufacturer’s specifications and installation instructions for each type of access panel required.

  - QUALITY ASSURANCE

    - Fire-Resistance Ratings: Wherever a fire-resistance classification is shown, or for construction where access panels are installed, provide a complete access door assembly as required with panel door, frame, hinge and latch from a manufacturer listed in the Underwriters Laboratories, Inc., “Classified Building Materials Index” for the rating shown. Provide a UL label on each access panel.

    - Number, locations and sizes of panels shown on the drawings are approximate. Coordinate with the mechanical subcontractor and submit drawings verifying locations and sizes of access panels. Indicate thickness and door material.

    - Size Variations: Obtain the Engineer’s approval of the manufacturer’s standard-size units which may vary slightly from the sizes noted.

- PRODUCTS

A.	If only one product is acceptable (single or sole source product), obtain an approved Competition Waiver and submit to the CPO Construction, Contract Administrator.  The language shall read as: “Manufacturer Name, Product # XXXXX, No Equal.”  Refer to CPO-6 Competition Waiver Policy for more information.

B.	If a Competition Waiver is not approved or more than one product is acceptable, this section must list a minimum of 2 products plus the language “Or Approved Equal,” along with salient characteristics.  Refer to CPO Construction’s Salient Characteristics Guidelines for more information.

  - MATERIALS AND FABRICATION

    - General: Provide an access panel assembly manufactured as an integral unit, complete with all parts and ready for installation. Provide the manufacturer’s standard insulated, flush panel door with continuous piano hinge and self-closing mechanism.

    - Locking Devices: Provide one cylinder lock per access door. Furnish two keys per lock and all locks alike. See Section 08 70 00 – Hardware.

    - Plastic laminate facing material shall match adjacent plastic laminate paneling.

- EXECUTION

  - INSTALLATION

    - Comply with the manufacturer’s instructions for installation of access panels.

    - Coordinate installation with the Work of other trades.

    - Set frames accurately in position and securely attach to supports with face panels plumb or level in relation to adjacent finish surfaces.

    - Adjust hardware and panels, after installation, for proper operation.

    - Remove and replace panels or frames which are warped, bowed, or otherwise damaged.

- MEASUREMENT AND PAYMENT

  - GENERAL

    - No separate measurement or payment will be made for the Work required by this section. The cost for this portion of the Work will be considered incidental to, and included in the payments made for the applicable bid items in the [Schedule of Unit Prices] [Lump Sum price bid for the Project].

End of Section

Revision History:

05/01/2014 Conversion to 2004 CSI Numbering System

10/15/2014 Added Sole Source and Salient Characteristics Note to Part 2