Project Name:____________________________________
Project Number: __________________________________

Contractor Name: _________________________________

Inspector Name:___________________________________

Inspector Signature:______________________________________

Date:_____________________________

|  |  | Confirm |
| --- | --- | --- |
| 1 | Verify that the proper connector has been provided for the operating voltage and to accommodate the cable sizes and types used. | Compliant Non-Compliant NOTES: |
| 2 | Verify the insulation has been stripped back to the proper specified dimension for each layer and type of insulation in preparation for connector installation, in accordance with manufacturer’s instructions. | Compliant Non-Compliant NOTES: |
| 3 | Verify that the stripped cable has been properly cleaned in preparation for connector installation. Verify that the semiconductor material residue has been cleaned off the insulation using an axial wiping motion to eliminate tracking. | Compliant Non-Compliant NOTES: |
| 4 | Verify that the compression lug or connector supplied with the connector kit has been properly crimped per the manufacturer’s written instructions | Compliant Non-Compliant NOTES: |
|  | Verify that the connector is torqued to proper values per the manufacturer’s published torque values. | Compliant Non-Compliant NOTES: |